# IOTA -	a code to study ion transport and radiation damage in composite materials

C.H.M.Broeders, A.Yu.Konobeyev, K.Voukelatou

#### February, 2004

Summary

The code IOTA has been elaborated to study primary radiation defects in composite materials irradiated by ions.

The code may be used for the calculation of the total number of primary defects created in materials, for the displacement cross-section and for the spatial defect distribution. The calculations are based on the use of tabulated data or of analytical expressions for the differential cross-section for the transfer of the kinetic energy from the moving ion to a lattice atom. The simulation is based on the binary collision approximation and the Monte Carlo method.

Using the results obtained with the help of the method of molecular dynamics (MD), the IOTA code may also be applied for high energy calculations in joint BCA-MD calculations.

Zusammenfassung

Der Code IOTA wurde entwickelt mit dem Ziel primäre Schäden durch Bestrahlung mit Ionen in Verbundwerkstoffe untersuchen zu können.

Der Code kann benützt werden für die Berechnung der Gesamtzahl der primären Schäden in den Materialien, für Displacement Wirkungsquerschnitte und für die räumlichen Verteilungen der Strahlenschäden. Die Berechnungen basieren auf die Anwendung von tabellierten Daten oder von analytischen Formeln für die differentiellen Wirkungsquerschnitte für den Transfer der kinetischen Energie des bewegenden Teilchens zum Atom im Gitter. Die Simulation basiert auf die „binary collision approximation“  (BCA) und auf Monte Carlo Verfahren.

Unter Verwendung von Ergebnisse von Rechnungen nach der Methode der Molekular Dynamik (MD) kann der IOTA Code auch für hochenergetische Rechnungen in gekoppelten BCA-MD Rechnungen angewandt werden.

CONTENTS

page

1. Specific features of the code ………………………………………………		1

2. Brief description of the method of calculations ……………………………		2

##### 2.1	Monte Carlo simulation ……………………………………………		2

##### 2.2	Integration ………………………………………………………….		6

##### 2.3	Simple evaluation ………………………………………………….		8

##### 2.4	Combined BCA-MD calculation………………………………….		8

##### 2.5	Flowchart of the code……………………………………………...		9

3. Computer compatibility ……………………………………………………	10

4. Execution of the IOTA code ………………………………………………	10

5. Preparation of the main input file for the IOTA code …………………….	11

6. Examples of the INPUT file ………………………………………………..	16

7. Preparation of the data for the stopping power ……………………………	19

8. Description of the output file of the IOTA code …………………………..	22

9. Example of the calculation …………………………………………………	29

#### References …………………………..………………………………………..	30

Description of the diskette with the IOTA code distributed with this report. Windows version using GNU FORTRAN Compiler …….………………….	31

The text of the IOTA code ……………………………………………………	32