The code IOTA (IOn TrAnsport) has been created for the investigation of the primary radiation defects in composite materials irradiated by ions.

The code calculates the total number of primary defects created in materials, the displacement cross-section and the spatial defect distribution. The calculations are performed with the help of the different approaches described below. The simulation of the ion movement in the media is based on the binary collision approximation (BCA) and the Monte Carlo method.

The IOTA code can be applied for high energy calculations using the results obtained with the help of the method of molecular dynamics (joint BCA-MD calculations).

History.  The creation of the IOTA code was initiated in January, 2003 and the first version of the code was approved in April, 2003. The current version is realized in December, 2003.

The service code SL_LNS4A used for the input data preparation was written in 2000. Now it was checked and rewritten.

1. Specific features of the code

The IOTA code can be applied for composite materials having up to 21 different components. The code is based on the use of any justified and approved differential cross-section d(E,T)/dT [1-3] for the transfer of the kinetic energy T from the moving ion to the lattice atom.

The IOTA code does not repeat any parts and algorithms of the popular codes based on the BCA approach [4-7].

The advantages of the IOTA code are

i) the possibility to simulate the damage process based on the different approaches with the help of the Monte Carlo method, and by the numerical integration of the corresponding distributions in a single computer run;

ii) the algorithm allows easily incorporation of the results obtained by the MD method and to perform  combined BCA-MD calculations;

iii) the faster running time comparing with the other BCA codes.

2. Brief description of the method of calculations

For the incident ion with kinetic energy E the radiation damage dose rate is defined by the particle flux and the atomic displacement cross-section d(E). This cross-section is calculated as follows

σd(E)=dσ(E,T)dTν(T)dT,	(1)

where d(E,T)/dT is the differential cross-sections for the transfer of a kinetic energy T from the incident ion to a primary knocked-on atom (PKA), (T) is the cascade function equal to the total number of displaced atoms produced by the PKA of kinetic energy T.

The IOTA code calculates the value of the cascade function (T), the space distribution of the defects created and the displacement cross-section d(E).

A single computer run for the incident energy E gives the (T) values for the whole energy range below this energy.

##### 2.1 Monte Carlo simulation

The simulation of the ion transport can be described by the following scheme.

1.	 Firstly the mean free path L of the primary ion is calculated with the help of the expression

L=1σint(E) N0,	(2)

where E is the kinetic energy of the projectile, N0 is the number density of atoms in a material.

The cross-section int(E) in Eq.(2) corresponds to the interaction of the primary ion with the lattice atom and the transfer to it of the portion of the kinetic energy exceeding the threshold displacement energy Ed. It is calculated as follows

σint(E)=i=1Nwi σint(i)(E) =i=1NwiEd(i)Tmax(i)dσi(E,T),	(3)

where wi is the atomic fraction of the i-th component of the target material, Ed(i) is the effective threshold displacement energy for i-th component, Tmax(i) is the maximal energy transferred from the ion to the PKA defined by the kinematics.

In the present version of the IOTA code the differential cross-section for the transfer of the kinetic energy T from the primary ion to the lattice atom d(E,T) is calculated as follows

dσi(E,T)=πa2f(t12)dt2t32,		(4)

where the function f(t1/2) and the screening length “a” are calculated based on the different approaches depending on the user choice by input.

i) The f(t1/2) function and screening length are defined according to Ref.[2] (the IOTA code parameter IDSDT on Card 2 with valid values 0, 10-15 from Table 1)

f(t12)=λ t12−m 1+(2λ t1−m)q−1q,

a=a0(9π2128)13Z123+Z223−12,

where the corresponding available sets of the , m and q parameters are listed in Table 1.

Table 1

Sets of the , m and q parameters for the f(t1/2) function calculation and the IOTA code parameter IDSDT from Ref.[2]

|  | M | q | Reference | IDSDT |
| --- | --- | --- | --- | --- |
| 1.309 | 1/3 | 2/3 | [2] | 0 or 10 |
| 1.7 | 0.311 | 0.588 | [13] | 11 |
| 2.37 | 0.103 | 0.570 | [13] | 12 |
| 2.92 | 0.191 | 0.512 | [13] | 13 |
| 3.07 | 0.216 | 0.530 | [13] | 14 |
| 3.35 | 0.2328 | 0.4445 | [14] | 15 |

ii) The f(t1/2) function and ‘a” are calculated according to Ref.[3]

(IDSDT =1 on Card 2)

f(t12)=a1tj4x+a2(x+a3)2

j=1, a1=0.088, a2=1, a3=0.2	for t1/2  0.04

j=1, a1=0.23, a2=1.3, a3=0.4	for 0.04 < t1/2  1

j=0, a1=0.4563, a2=0, a3=0.3	for 1 < t1/2  40

j=0, a1=0.5, a2=0, a3=0		for t1/2 > 40

a=a0(9π2128)13Z112+Z212−23,

For both cases i) and ii)

t=ε Tα2ε0,

ε=Eε0=a A2 EZ1 Z2 e2(A1+A2),

α2=4M1M2(M1+M2)2

where Z1, A1 and Z2, A2 are the atomic number and mass number of the projectile and the target material, respectively, a0 is the Bohr radius.

The values of the int(E) cross-section are calculated by the SL_LNS4A code before the IOTA code run and stored on special files.

2.	The trajectory of the ion is divided in small steps. The step l is defined as a minimal value from the part of the mean free path L/n and the distance x, where the ion energy loss is less than the part of the ion kinetic energy E. The values of parameters “n” and  adopted in the current version of the code are equal to 10 and 0.02 correspondingly. The value of x is obtained from the total stopping power.

3.	For the defined l value the elastic interaction of primary ion with a lattice atom is simulated by Monte Carlo. The elastic interaction corresponds to the transfer of the energy to PKA more than the threshold displacement energy Ed.

4.	If the elastic event occurs, the type of the PKA from the composite material is defined by the stochastic method based on wi(i)int values (Eq.(3)). Than the kinetic energy T of the PKA is obtained with the help of the d(E,T) distribution (Eq.(4) by the Monte Carlo “rejection” method.

5.	The primary ion passes the distance l and losses the energy transferred to the PKA and for the electronic excitation. The last energy is defined from the electronic stopping power (dE/dx)el.

The PKA characteristics are stored for the further treatment.

The new mean free path is defined for the primary ion and the procedure described above is repeated.

6.	If the elastic collision with the energy transfer exceeding Ed does not occur, the primary ion moves on the distance l. Its energy is decreased corresponding to the experimental electronic stopping power (dE/dx)el and the part of the nuclear stopping power dEdxn0−Ed, which corresponds to the energy transfer from the ion to the lattice atom from 0 to Ed value. The data dEdxn0−Ed are prepared by the SL_LNS4A code before the IOTA code run.

7.	The primary ion is tracing while its kinetic energy is less than the minimal effective threshold displacement energy Ed(i) from all N-components of the composite material.

8.	The stored PKA’s are treated as primary ions producing new knock-on atoms and procedure described above repeats.

9.	The process described above is repeated until the energy of all PKA’s created is less than the minimal Ed energy and the computer bank of PKA’s is empty.

##### 2.2 Integration

Along with the Monte Carlo simulation the IOTA code calculates the number of the primary defects (E) by the numerical integration of the ratio of the losses for damage production to the total stopping power. The total number of Frenkel pairs produced by the ion with the energy E0 in the material is calculated using the following expression [1]

ν(E0)=0.82Ed T(E0)= 0.82Ed 0E0(dEdx)dam(dEdx)el+(dEdx)ndE ,	(5)

where E0 is the primary ion energy, T is the energy going to the production of displaced atoms.

The value (dE/dx)dam in Eq.(5) is the specific energy loss for the damage production, (dE/dx)el is the electronic stopping power and (dE/dx)n is the specific energy loss for the elastic scattering (nuclear loss).

The specific energy loss for the damage production is calculated as follows

dEdxdam=N0EdTmaxdσ(E,T)dTT(T) dT,	(6)

where N0 is the number density of atoms in a material, T(T) is the energy for defects production, d(E,T) is calculated by Eq.(4).

The energy for the damage production T(T) is calculated according to the NRT-approach [4,8]

TNRT(T)=T1+k g(ε)	(keV), 	(7)

g()=3.4008 1/6 + 0.40244 3/4 + ,

k=323π meM212(A1+A2)32Z123Z212A132(Z123+Z223)34,

ε=A2T(A1+A2) a(Z1Z2e2),

where me is the mass of an electron, M2 is the mass of the atom of target material and the other values are defined above, e is the electron charge.

Note, that the value of “k” in Eq.(7) is defined according to Robinson [4] and for the case of Z1=Z2, A1=A2 it coincides with the NRT-value [8] equal to k=0.1337 Z11/6 (Z1A1)12.

##### 2.3 Simple evaluation

For the comparison with other calculations, the value of the cascade function (E) is evaluated with the help of the NRT-formula [8]

ν(E)NRT=0.82Ed TNRT (E),	(8)

where TNRT(E) is calculated by Eq.(7).

##### 2.4 Combined BCA-MD calculation

The simulation described in Sect.2.1 is performed up to a critical ion energy Tcrit, below this energy the results of MD calculations are used to obtain the total number of defects. This energy is applied as for the primary ion, as for PKA’s produced in the collision cascade with the energy above Tcrit.

The typical value of Tcrit is between 5 keV and 100 keV depending on the MD simulation.

The results of the MD calculations are introduced in the code in the form of the defect production efficiency. The efficiency is equal to the ratio of the total number of survived point defects (Frenkel pairs) to the number of defects calculated by the NRT model (Eq.(8)).

The general expression for the defect production efficiency is as follows

η(EMD)=α1 EMDα2 + α3 EMD,	(9)

where the energy EMD is supposed equal to the TNRT energy in Eq.(7), i are fitting coefficients.

To perform the calculation the user should supply the values of the i  parameters in Eq.(9) or use the default values which are available in the code for a limited number of elements (Fe, Cu, W).

##### 2.5 Flowchart of the code

The flowchart of the code is shown in Fig.1

User input data

Interaction cross-sections prepared by SL_LNS4A code

Stopping power data

(Chapter 7)

Preparation data for calculations

Basic Monte Carlo calculations

(Chapter 2.1)

Integration

(Chapter 2.2)

Evaluation and printing results

Fig.1

3. Computer compatibility

The IOTA code and the service code SL_LNS4A are written in FORTRAN.

The IOTA code uses a random number generator, which can be different for different FORTRAN compilers. The current version of the IOTA code uses the GNU FORTRAN [9] built-in random number generator RAND. To substitute it by the other one it is necessary to check the FUNCTION RANDOM in the IOTA code.

4. Execution of the IOTA code

The execution of the IOTA code includes several steps.

1.	The preparation of the main input file. The name is INPUT. The procedure is described below in Sect.5.

2.	The stopping power calculation by the SRIM code [7] if the option ‘SRIM’ is chosen in the input file (Sect.5). The details are discussed in Sect.7.

If the ‘SPAR’ option is selected in INPUT, the stopping power is calculated by the IOTA code and point 2. is omitted.

3.	Run SL_LNS4A code. The input file is the same as for the IOTA code (the file INPUT). After the SL_LNS4A code execution the files with the information needed for the IOTA code run are created. After the file creation it is not necessary to execute this code again for the other IOTA code runs with the same INPUT and other set of the primary ion energies, which do not exceed the TLIM value (Sect.5).

4.	Run the IOTA code. Analysis of the results.

5. Preparation of the main input file for the IOTA code

The main input file is called INPUT. It is located in the same directory with the executable modules of the IOTA code and the SL_LNS4A code.

###### The file INPUT

All data are introduced in the file in the free format form including the names of the external files. Each line beginning from the symbol ‘*’, ‘c’ or ‘!’ defines the comment card. The length of the external file names should not exceed 12 characters.

The structure of the INPUT file is described below.

(Card 1) The title of the task described in 80 characters including blank spaces.

(Card 2) Parameters IGRAPH, MAXPLAY, TLIM, IDSDT.

Parameter IGRAPH is the damage profile printing option

. The value IGRAPH = 0 suppresses the printing of the damage profiles (space damage distribution).

IGRAPH = 1 means the printing.

The parameter MAXPLAY defines the maximal number of attempts to get the kinetic energy T of PKA from the d(E,T)/dT distribution in the FUNCTION TPKA. The play of T value is performing by the Monte Carlo “rejection” method. If after MAXPLAY attempts the definition of T failed, the kinetic energy of PKA is taken according to the ratio

Tmean=EdTmaxdσ(E,T')dT'T' dT' /EdTmaxdσ(E,T') ,	(10)

the Tmean values are calculated by the SL_LNS4A code before the IOTA code run.

The possible values of MAXPLAY are

- actual number of attempts

- MAXPLAY=1. In this case the kinetic energy T of PKA is substituted by Tmean from Eq.(10). It greatly reduces the time of the calculation at the high ion energies, but should be referred to a approximate calculations

- MAXPLAY=0.  It means the use of the default value of MAXPLAY equal to 250,000

- MAXPLAY < 0 means the unlimited number of attempts. It incredibly increases the running time of the code at the high energies of the primary ions.

The value of TLIM defines the maximal energy (MeV) to be treated by the SL_LNS4A code. It should exceed the maximal energy of the primary ion introduced in the IOTA code input file (see below). The default value (TLIM=0) is equal to 5 GeV.

Parameter IDSDT defines the method of the f(t1/2) function calculation (Eq.(4)). The values IDSDT=0, 10-15 correspond to the f(t1/2) function from Ref.[2] (see Table 1 for IDSDT definition). The value IDSDT=1 means the calculation of f(t1/2) with the help of the approach from Ref.[3].

(Special cards 1’ and 2’) These cards are used only for the combined BCA-MD calculations.

[Card 1’] To initiate the calculations the card should contain the words ‘BCA’ and ‘MD’ in any combination printed in the same line (See Example 3)

[Card 2’] CMD(1), CMD(2), CMD(3), ELOW, EHIGH, ECRIT

The CMD(i) values correspond to the coefficients i in Eq.(9) for the efficiency of the defect production obtained from the MD calculations.

ELOW and EHIGH are the lowest and the highest EMD (or TNRT) energy in keV, where the efficiency supposed to be a constant,  (EMD < ELOW) = (ELOW),  (EMD > EHIGH) = (EHIGH).

The energy ECRIT (keV) is equal to the value of the EMD energy in Eq.(9) where the MD results for the defect production are used instead of the BCA calculations (Sect.2.4). Usually, ECRIT is equal to EHIGH. The critical energy Tcrit mentioned in Sect.2.4 is calculated by the code basing on the ECRIT value for the incident ion and the ions of the target material.

The default values are taken if CMD(i),i=1,3; ELOW; EHIGH and ECRIT are set to zero. They are available for Fe, Cu and W in the current code version

(Card 3) The atomic number (Z) and the atomic weight (A) (amu) of the projectile.

If A=0 the atomic weight is taken for the natural mixture of isotopes for this element (Z) from internal code tables.

(Card 4) The density of the target composite material (RO), the number of the components of the compound (NK) and the stopping power option (SO).

The RO value is introduced in g/cm3. If RO=0, the density is taken from the internal code table for this material.

The NK value is equal to the number of unique atoms of the target. For example, the NK value is equal to 2 for Li2O, and equal to 3 for Al2(SO4)3. It is for the case if only the single isotope of each element is considered in the calculation. For the mixture of the isotopes the NK value should include their amount. If the lithium from the Li2O compound is considered as a mixture of 6Li and 7Li isotopes, the NK value is equal to 3.

The stopping power option (SO) is a character variable describing the method of the stopping power calculation. It can be equal as follows

i) ‘SRIM’ (or ‘Ziegler’).  The data for the stopping power are taken from the special files prepared by the SRIM code [7]. (See Card 6 ,9 and Sect.7). This option is recommended.

ii) ‘SPAR’ (or ‘Armstrong’).  The stopping power is calculated in the IOTA code by the SPAR routine [15,16]. It should be noted that for the heavy ions and high energies the accuracy of this method is less than of the SRIM code.

(Card 5, 6 or more if necessary) Names of files with the cross-sections and other information to be prepared by the SL_LNS4A code. Each name corresponds to the interaction of the primary ion with the certain material component. For example, for an -particle projectile and Li2O target the names are introduced for +O and +Li interactions.

The sequence of the file names should be the same as for the description of the components of the composite material (Card 7 and others). The names are separated by at least one blank space. Any number of cards can be used for the total list of the file names.

(Card 6) Name of file with stopping power data corresponding to the primary ion interaction with the compound (e.g. with Li2O). The format of the file corresponds to the SRIM code output [7] (see Sect. 7).

If the ‘SPAR’ option is selected for the stopping power calculation (Card 4) the Card 6 should be omitted.

(Card 7) The atomic number (Z), the atomic weight (A) (amu), the atomic concentration (W) and the effective threshold displacement energy (ED) for the first component of the composite material (target).

If the zero value is introduced instead of A value (A=0), the atomic weight is taken for the natural mixture of isotopes with the atomic number Z from the internal code table.

The atomic concentration (W) is introduced in the normalized or the un-normalized form. For example, in the description of the oxygen from Li2O compound both values 0.333333 or 1 are correct.

The displacement energy (ED) is introduced in eV. The default value (ED=0) is equal to 40 eV.

(Card 8, 9 or more if necessary) Names of files with the data to be prepared by the SL_LNS4A code for the interaction of the ion described in Card 7 with each component of the composite material. For example, if the Card 7 describes the oxygen from Li2O, the current card should include the file names for the O+O and O+Li interactions.

Note, the sequence of such file names should coincide with the sequence of the different material components description (given in the cards similar to the Card 7).

(Card 9) Name of file with the stopping power data from the SRIM code corresponding to the interaction of the ion describing in the Card 7 with the compound. For example, if the Card 7 describes the oxygen from Li2O compound, the name of file given here is for the stopping power data for the O+Li2O interaction.

This card is omitted for the ‘SPAR’ option chosen for the stopping power calculation in Card 4.

(Next cards similar to Cards 7-9). The same for other components of the compound. For example, if the oxygen from Li2O is described in Cards 7-9 these cards are used for the lithium description.

(Card after the material description).  Names of the output files.

Two file names are introduced for the usual calculations and three file names for the joint BCA-MD calculations.

The first output file contains the detail information about the calculation performed. The second file repeats partly the data from the main output for the special use, e.g. for the graphics representation. The third file contains the results of the combined BCA-MD simulation.

(Next cards). The energy of primary ion (E) and the number of Monte Carlo events to perform the calculation at this energy (NHIST).

The energy is introduced on the separate cards following one after another in MeV along with the unique NHIST number.

The negative value shown in the list is considered as the last energy in the simulation. For example, to perform the calculation for incident energy equal to 10 keV with 999 Monte Carlo events and the primary energy equal to 1 GeV with 100 Monte Carlo events, one should introduce two successive cards 0.010  999 and –1000.0  100

6. Examples of the INPUT file

A)	Use the ‘SRIM’ option for the stopping power calculation

The example of the INPUT file for the interaction of 28Si with the silicon carbide is given below (Example 1)

###### Example 1

###### The INPUT file for Si+SiC interactions with the stopping power from SRIM code

* IOTA code input

* name of the task

Test calculations    Si+Silicon Carbide

1  0  10.  0

* Z, A of the projectile ion

14  28

* density (g/cm3), number of components and stopping option

3.21    2   SRIM

* name of files with the cross-sections from SL_LNS4A code

Si_Si.dat    Si_C.dat

* dE/dx for projectile + composite media

Si_SiC.Zie

*

*  description of the components for compound

* Z,  A,  atomic fraction, Ed (eV)

14  28   0.5000   40.0

* cross-sections

Si_Si.dat   Si_C.dat

*   dE/dx

Si_SiC.Zie

6  12   0.5000   20.0

C_Si.dat    C_C.dat

C_SiC.Zie

*

* names of output files

messages  mess2.dat

*

*  energy of projectile and number of Monte Carlo events

0.010  1000

0.1    1000

-1.0    1000

B)	Use the ‘SPAR’ option for the stopping power calculation

The example of the INPUT file is given here (Example 2).

###### Example 2

###### The INPUT file for Si+SiC with the stopping power from the SPAR calculations

* IOTA code input

* name of the task

Test calculations    Si+Silicon Carbide

*

1  0  10.  0

* Z, A of the projectile ion

14  28

* density (g/cm3), number of components and stopping option

3.21    2   SPAR

*

* name of files with the cross-sections from SL_LNS4A code

Si_Si.dat    Si_C.dat

*

*  description of the components for compound

* Z,  A,  atomic fraction, Ed (eV)

14  28   0.5000   40.0

* cross-sections

Si_Si.dat   Si_C.dat

*

6  12   0.5000   20.0

C_Si.dat    C_C.dat

*

* names of output files

messages  mess2.dat

*

*  energy of projectile and number of Monte Carlo events

0.010  1000

0.1    1000

-1.0    1000

C)	Combined BCA-MD calculations

The example of the INPUT file for the irradiation of natural tungsten by 167Tm ions is given below (Example 3). The stopping power is obtained by the SRIM code. The default parameters are used for the efficiency calculations at low energies.

###### Example 3

###### The INPUT file for joint BCA-MD calculation for Tm+W interaction

* IOTA code input

* name of the task

Test calculations    Tm+W

0 700000 500.1  0

*

BCA + MD

0  0  0  0  0  0

* Z, A of the projectile ion

69.  167.

* density (g/cm3), number of components and stopping option

0    1    Ziegler

* name of files with the cross-sections from SL_LNS4A code

tm_w.dat

* dE/dx for projectile + media

tm_w.zie

*

* Z,  A,  atomic fraction, Ed (eV)

74  0   1.0000   90.0

* cross-sections

w_w.dat

* <  dE/dx   >

w_w.zie

*

* names of output files

messages  mess2.dat  bcamd.out

*

*  energy of projectile and number of Monte Carlo events

0.01   100

1.0    100

10.   100

100.   50

-500.  50

*End

7. Preparation of the data for the stopping power

This section discusses the preparation of the stopping power data with the help of the SRIM code. It corresponds to the recommended input option ‘SRIM’ or ‘Ziegler’ (see Sect.5, Card 4) for the stopping power calculation.

If the ‘SPAR’ option is chosen in Card 4 of the INPUT file this section should be omitted.

The current version of the IOTA code works with the output format of the SRIM code [7]. The code is distributed by the author [7] freely.

To use the SRIM code one should download it from Ref.[7] and install it on PC.

If the SRIM code is installed, the instruction to prepare the data for the stopping power suitable for the IOTA code calculation is the following.

1. Select “Stopping/Range Tables” in the main SRIM menu (general panel).

2. Select the appropriate projectile and the target material. In the case of the compound one should introduce the correct density of the compound. It is the same as in the Card 4 of the INPUT file of the IOTA code (see Sect.5).

3. Set the energy range of the projectile from 1 eV to 5.2 GeV (the maximal energy allowable in the IOTA code).

4. Select the units for the stopping power equal to MeV/(mg/cm2)) (default in the main SRIM menu).

5. Perform the operation “Calculate table” and give the same name for the output file as shown in the file INPUT for the IOTA code (Sect.5). Store the output file in the same directory where there are the executable modules of the SL_LNS4A code and the IOTA code.

6. Repeat this procedure for all atoms of the compound, considered as a projectile.

The example of the output of the SRIM code for the case of Si+SiC interaction is given below (Example 4).

Note. If the calculations are performed for the same material and the various projectiles with the equal atomic number and the different atomic weights it is not necessary to prepare the SRIM stopping power data for each isotope.

If the data are prepared for the ion with Z and A and the actual projectile in the task is Z and A’, the electronic stopping power is correctly recalculated for a new energy grid (A’/A)E. The nuclear stopping power is renormalized according to the following formula

(dEdx)nZ,A'(dEdx)nZ,A = A'A −t' 1+t' +ln(t' +1+t') −t 1+t +ln(t +1+t),

t'= 1.948⋅104 E ATZ ZT(A'+AT)(Z23+ZT23)1289,

t= 1.948⋅104 E ATZ ZT(A+AT)(Z23+ZT23)1289,

where E is the energy of projectile (MeV), ZT and AT are the atomic number and the atomic weight of the target material.

The formula shown above is obtained from the analytical integration of Td(E,T)/dT, where d(E,T) is calculated by Eq.(4) and the f(t1/2) function is defined by the , m and q parameters from the first line of Table 1.

New data for the stopping power are written in the output file RENORM.STO.

Example 4

The SRIM code output for the S+SiC interaction

==================================================================

Calculation using SRIM-2003

SRIM version ---> SRIM-2003.10

Calc. date   ---> March 24, 2003

==================================================================

Disk File Name = SRIM Outputs\Si_SiC.Zie

Ion = Silicon [14] , Mass = 27.977 amu

Target Density =  3.2100E+00 g/cm3 = 9.6419E+22 atoms/cm3

======= Target  Composition ========

Atom   Atom   Atomic    Mass

Name   Numb   Percent   Percent

----   ----   -------   -------

Si     14    050.00    070.05

C      6    050.00    029.95

====================================

Bragg Correction = 0.00%

Stopping Units =  MeV / (mg/cm2)

See bottom of Table for other Stopping units

Ion        dE/dx      dE/dx     Projected  Longitudinal   Lateral

Energy      Elec.      Nuclear     Range     Straggling   Straggling

-----------  ---------- ---------- ----------  ----------  ----------

1.1 eV    6.406E-03  3.765E-02       1 A         1 A           A

1.2 eV    6.406E-03  3.986E-02       1 A         1 A           A

1.3 eV    6.667E-03  4.201E-02       1 A         1 A           A

1.4 eV    6.919E-03  4.408E-02       1 A         1 A           A

1.5 eV    7.162E-03  4.611E-02       1 A         1 A           A

1.6 eV    7.397E-03  4.808E-02       1 A         1 A           A

1.7 eV    7.624E-03  5.000E-02       1 A         1 A           A

1.8 eV    7.845E-03  5.187E-02       1 A         1 A           A

2 eV    8.270E-03  5.551E-02       1 A         1 A         1 A

2.25 eV    8.771E-03  5.985E-02       1 A         1 A         1 A

2.49999 eV    9.246E-03  6.400E-02       1 A         1 A         1 A

2.74999 eV    9.697E-03  6.799E-02       1 A         1 A         1 A

2.99999 eV    1.013E-02  7.182E-02       2 A         1 A         1 A

3.24999 eV    1.054E-02  7.553E-02       2 A         1 A         1 A

3.49999 eV    1.094E-02  7.912E-02       2 A         1 A         1 A

LINES SKIPPED

1.80 GeV   1.658E+00  6.462E-04    1.94 mm    69.09 um    12.42 um

2.00 GeV   1.529E+00  5.872E-04    2.33 mm    88.62 um    14.79 um

2.25 GeV   1.398E+00  5.276E-04    2.86 mm   116.43 um    17.99 um

2.50 GeV   1.291E+00  4.794E-04    3.44 mm   142.51 um    21.45 um

2.75 GeV   1.203E+00  4.395E-04    4.07 mm   167.81 um    25.16 um

3.00 GeV   1.128E+00  4.060E-04    4.73 mm   192.74 um    29.11 um

3.25 GeV   1.065E+00  3.774E-04    5.44 mm   217.50 um    33.28 um

3.50 GeV   1.009E+00  3.527E-04    6.20 mm   242.21 um    37.66 um

3.75 GeV   9.611E-01  3.312E-04    6.99 mm   266.93 um    42.24 um

4.00 GeV   9.186E-01  3.122E-04    7.82 mm   291.70 um    47.03 um

4.50 GeV   8.471E-01  2.803E-04    9.58 mm   384.34 um    57.15 um

5.00 GeV   7.893E-01  2.546E-04   11.49 mm   469.70 um    67.95 um

5.20 GeV   7.702E-01  2.456E-04   12.29 mm   483.24 um    72.45 um

4.50 GeV   8.471E-01  2.803F-04    9.58 mm   384.34 um    57.15 um

5.00 GeV   7.893F-01  2.546E-04   11.49 mm   469.70 um    67.95 um

5.20 GeV   7.702E-01  2.456E-04   12.29 mm   483.24 um    72.45 um

-----------------------------------------------------------

Multiply Stopping by        for Stopping Units

-------------------        ------------------

3.2099E+01                 eV / Angstrom

8. Description of the output file of the IOTA code

The following information is printed in the main output listing of the IOTA code. See Example 5.

- The name of the exercise introduced in the first non-comment card of the INPUT file (Sect.5).

- The characteristics of the primary ion and the target material (Primary particle (Z,A) Material at % Ed (eV)).

- The density of the material and the number of atoms in cm3 (n0).

- The effective Z and A numbers for the compound (Z(eff) and A(eff)) calculated as the sum of Z and A for all components of the composite material weighted with their atomic fractions. The values Z(eff) and A(eff) are used for the crude estimation of the number of defects created in material with the help of the NRT-formula.

- The energy of the projectile in MeV (Primary energy).

- The number of Monte Carlo simulations for the incident ion energy (Number of events).

- The number of primary defects calculated with the help of the NRT-formula (Eq.(8),(7)) (Simple NRT-formula prediction). For the composite materials the effective values Z(eff) and A(eff) (see Sect.”d”) are used instead of Z2 and A2 in Eq.(7).

- The number of primary defects calculated for all successive elastic interactions of the primary ion with the lattice atoms simulated by Monte Carlo (Number of vacancies (Lindhard, free path)). For the single interaction of the ion with kinetic energy E with the lattice atom the number of defects is estimated by the following expression

Δν=0.82EdEdTmaxdσ(E,T')dT'T(T') dT' /EdTmaxdσ(E,T') ,	(10)

where the energy for damage production T(T) is calculated according to Eq.(7). The T(T) value is defined using the effective Z(eff) and A(eff) numbers for the compound.

The statistical error (%) (in the Example 5 it is equal to 1.02%), the ratio to the number of defects calculated by the NRT-formula (0.667) and the reference number (2) are printed on this line.

- Number of defects calculated by the integration of Eq.(5) (Number of vacancies (Lindhard, integration)). The reference number is equal to (3).

- Number of defects (Frenkel pairs) calculated by the direct counting of all PKA’s created (Number of Frenkel pairs (direct counting)). It is a main characteristic obtained. Method of the calculation is described in Sect.2.1. The statistical error (%), the ratio to the number of defects obtained by the NRT-formula and the reference number (4) are also printed.

- The value described above in Sect.”j” multiplied on the coefficient equal to 0.8 (Modified counting).

- The number of defects calculated with the help of the NRT formula (Eq.(8),(7)) for each component of the compound and weighted with their atomic fractions (NRT-formula for pure material components weighted).

Such estimation repeats widely used and rather incorrect counting of the number of defects for compounds. Here the total number of defects is equal to

ν(E)=i=1Nwi νNRT(Z1,A1,Z2(i),A2(i))	(11)

where the sum is for all components of the composite material, wi is the atomic fraction, νNRT(Z1,A1,Z2(i),A2(i)) is calculated according to Eq.(8),(7) with the atomic and mass numbers Z2(i), A2(i) corresponding to the separate isolated i-th component of the compound.

- The approximate number of defects calculated (Number of vacancies from Tmean calc and Number of vacancies from Tplay calc). The values are of secondary importance.

- The displacement cross-section (d) for the incident ion energy calculated by the SL_LNS4A code in barns (Displacement cross-section (Lindhard, weighted)). The value of d is calculated as follows

σd(E)=i=1Nwi σd(i)(E)= i=1Nwi dσ(E,T,Z1,A1,Z2(i),A2(i))dTνNRT(T,Z2(i),A2(i),Z(eff),A(eff))dT,(12)

where d(E,T) is calculated by Eq.(4), the cascade function  is defined by Eq.(8),(7) with Z1 and A1 in Eq.(7) substituted by Z2(i) and A2(i) and with Z2, A2 in Eq.(7) substituted by Z(eff) and A(eff).

The separate non-weighted σd(i)(E) values are printed below in barns for each component of the composite material (Components:).

- The displacement cross-section (d) obtained from the direct counting of the displaced atoms by the Monte Carlo method (Sect.2.1) in barns (Displacement cross-section (direct counting)).

The value of d corresponds to the initial energy of the primary ion. It is calculated with the help of the following expression

σd(E)=ΔNdamϕ N0 ΔV

where Ndam is the number of PKA’s created in the volume V,  is the flux of the primary ions, N0 is the number of atoms per cm3.

The value of V is taken small enough to minimize the energy attenuation of the primary ion.

The statistical error (%) and the reference number (7) are also printed for this d value.

- The displacement cross-section (d) described in Sect.”o” multiplied on the coefficient equal to 0.8 (modified count).

- The average kinetic energy of PKA’s obtained with the help of different approaches (Tmean =…, Tplay=…)

- The range of the primary ion from the SRIM code tables (Path (Ziegler,evl)=) and the same value multiplied on the coefficient equal to (1+0.33333A2/A1) (total=). The last value has the physical meaning only at the low primary ion energies, where the energy loss is defined by the elastic collisions.

- The calculated range of the primary ion (Path (total,calc )). This is the approximate estimation of the range.

- The relative number (%) of calls of the FUNCTION TPKA, when the definition of the kinetic energy T failed after MAXPLAY attempts (T play was cut (TPKA rout)). The MAXPLAY value is also printed.

- The spatial distribution of dpa calculated basing on the approach described in Sect.”h” (Profile of damage from Lindhard dS/dx (free path )). The data are normalized on the primary ion fluence equal to 1/cm2. The units are cm for X-axis and dpa for the Y-axis. Note, that in the current version of the code “X” refers to the distance along the ion trajectory rather than for the distance counting from the plane normal to the primary momentum of the ion (z-axis).

- The space distribution of dpa calculated by the direct counting of PKA’s created (Profile of damage from direct counting). See other notes for Sect.”u”.

- The number of defects (T) at the different primary energies of the ion calculated below the initial ion energy E (Number of vacancies at different primary energies from current calculation).

The first column is the energy of the primary ion in MeV. The second column is the cascade function calculated by the approach described in Sect.”h”. The third column is for the number of defects obtained with the help of the integration procedure described in Sec.2.2. The last column shows the number of defects calculated from the counting of all PKA’s created. The columns have the reference numbers (2),(3),(4) corresponding to the type of the calculation described mentioned above in the IOTA listing.

The data shown in the table for each energy E(i) correspond to the average values for the range from 0.5(E(i) + E(i-1)) to 0.5(E(i+1) + E(i)). The values of the cascade function shown in the table have the maximal error at the small ion energies. To get the correct numbers of defects for such energies it is necessary to run the IOTA code for the same energies considered as the primary ones.

Example 5

Main output of the IOTA code

I O T A   C O D E

Test calculations    Si+Silicon Carbide

Primary particle (Z,A) 14  28

Material    at %    Ed (eV)

14  28    50.00     40.0

6  12    50.00     20.0

Density =  3.2100 g/cm3    n0= 9.66531E+22 1/cm3

Z(eff) = 10.00    A(eff)=  20.00

Note. Nuclear nonelastic interactions are not considered here

******************************************************************************

Primary energy   10.000     MeV

Number of events     100

error %      /NRT

Simple NRT-formula prediction               =  3454.1               1.000  (1)

Number of vacancies (Lindhard, free path )  = 2303.28  (  1.02)     0.667  (2)

Number of vacancies (Lindhard, integration) = 2407.12               0.697  (3)

----------------------------------------------------------------------------

Number of Frenkel pairs (direct counting)   = 3041.97  (  1.96)     0.881  (4)

----------------------------------------------------------------------------

Modified counting                           = 2433.58  (  1.96)     0.705

NRT-formula for pure material components weighted = 3354.17                (5)

Number of vacancies from Tmean calc               = 3159.10         0.915

Number of vacancies from Tplay calc               = 2412.17         0.698

Displacement cross-section (Lindhard, weighted) (b) = 1.12019E+07          (6)

Components:  1.30498E+07 9.35397E+06

Displacement cross-section (direct counting)    (b) = 1.43044E+07 ( 13.35) (7)

(modified count )    (b) = 1.14435E+07 ( 13.35)

Tmean = 3.10542E-01 MeV     Tplay= 0.32180

Path (Ziegler,evl)= 3.140E-04 cm,  total= 3.888E-04 cm

Path (total,calc )                      = 3.205E-04 cm

Histories interrupted (subr model)=           0

T play was cut (TPKA rout) =           1.030  %    Maxplay=   125000

Profile of damage from Lindhard dS/dx (free path )

X=(cm)  Y=(dpa), result is normalized on primary ion fluence 1/cm**2

X           Y           .........!.........!.........!.........!

0.00000E+00  0.00000E+00     I                  !                   I

1.94381E-05  1.21775E-17     *                  !                   I

3.88762E-05  1.27821E-17     *                  !                   I

5.83143F-05  1.38325E-17     *                  !                   I

7.77524E-05  1.55073F-17     I*                 !                   I

9.71905E-05  1.63149E-17     I*                 !                   I

1.16629E-04  1.97523F-17     I*                 !                   I

1.36067E-04  2.27565E-17     I *                !                   I

1.55505E-04  2.57695E-17     I *                !                   I

1.74943F-04  3.18961E-17     I--*---------------!-------------------I

1.94381E-04  3.76942E-17     I   *              !                   I

2.13819E-04  4.75221E-17     I    *             !                   I

2.33257E-04  6.29700E-17     I      *           !                   I

2.52695E-04  8.36897E-17     I         *        !                   I

2.72133F-04  1.15629E-16     I              *   !                   I

2.91571E-04  1.64671E-16     I                  !  *                I

3.11010E-04  2.60224E-16     I                  !               *   I

3.30448E-04  2.81363F-16     I                  !                   *

3.49886E-04  1.12125E-18     I                  !                   I

3.69324E-04  0.00000E+00     I------------------!-------------------I

3.88762E-04  0.00000E+00     I..................!...................I

Sum(Y)=  1.22596E-15 dpa    (unnormalized)=   2303.3

Profile of damage from direct counting

X=(cm)  Y=(dpa), result is normalized on primary ion fluence 1/cm**2

X           Y           .........!.........!.........!.........!

0.00000E+00  6.91949E-20     I                  !                   I

1.94381E-05  2.19986E-17     I*                 !                   I

3.88762E-05  2.01464E-17     I*                 !                   I

5.83143F-05  2.06680E-17     I*                 !                   I

7.77524E-05  2.14451E-17     I*                 !                   I

9.71905E-05  2.33293F-17     I*                 !                   I

1.16629E-04  2.92269E-17     I *                !                   I

1.36067E-04  4.10751E-17     I  *               !                   I

1.55505E-04  4.30126E-17     I  *               !                   I

1.74943F-04  4.21663F-17     I--*---------------!-------------------I

1.94381E-04  5.54996E-17     I    *             !                   I

2.13819E-04  7.39693F-17     I      *           !                   I

2.33257E-04  1.07625E-16     I         *        !                   I

2.52695E-04  1.30379E-16     I            *     !                   I

2.72133F-04  1.32343F-16     I            *     !                   I

2.91571E-04  1.97541E-16     I                  !*                  I

3.11010E-04  2.88415E-16     I                  !          *        I

3.30448E-04  3.64763F-16     I                  !                  *I

3.49886E-04  5.47172E-18     I                  !                   I

3.69324E-04  0.00000E+00     I------------------!-------------------I

3.88762E-04  0.00000E+00     I..................!...................I

Sum(Y)=  1.61914E-15 dpa    (unnormalized)=   3042.0

Number of vacancies at different primary energies from current calculation

[explanation (2),(3),(4) see above]

---------------------------------------------------------------------------

Energy     dS/dx(free path)   dS/dx(integr)     Direct counting

(MeV)          (2)               (3)               (4)

1.2500E-02      148.09            232.30            198.51

5.0000E-02      409.05            522.47            502.49

0.1000          596.70            711.68            702.07

0.1500          748.28            861.76            895.04

0.2000          874.16            983.53            1025.2

0.2500          981.76            1087.1            1145.3

0.3000          1071.0            1178.2            1247.0

0.3500          1147.6            1254.3            1330.7

0.4000          1214.8            1319.6            1424.8

0.4500          1275.1            1378.8            1507.0

0.5000          1328.7            1431.9            1591.3

LINES SKIPPED

8.750          2271.7            2374.9            2987.7

8.800          2273.3            2377.2            2992.5

8.850          2274.5            2379.5            2994.2

8.900          2275.7            2379.5            2995.3

8.950          2277.0            2381.8            2996.8

9.000          2278.5            2381.8            2998.6

9.050          2279.6            2384.1            2999.9

9.100          2280.7            2384.1            3001.8

9.150          2282.4            2386.4            3004.3

9.200          2282.9            2386.4            3005.0

9.250          2284.4            2388.7            3007.2

9.300          2286.3            2391.0            3010.2

9.350          2288.1            2391.0            3012.8

9.400          2289.5            2393.3            3015.0

9.450          2290.7            2393.3            3017.0

9.500          2291.9            2395.6            3020.4

9.550          2293.3            2395.6            3026.8

9.600          2294.5            2397.9            3032.7

9.650          2295.7            2397.9            3034.1

9.700          2297.6            2400.2            3035.9

9.750          2298.3            2400.2            3036.9

9.800          2299.4            2402.5            3038.0

9.850          2300.2            2402.5            3038.6

9.900          2301.3            2404.8            3039.8

9.950          2302.2            2404.8            3041.4

10.00          2303.3            2407.1            3042.0

*********************************e*n*d***********************************

9. Example of the calculation

Fig.2 and Table 2 show the examples of the calculations performed with the help of the IOTA code. For the comparison the result of the calculation obtained by the MARLOWE code in Ref.[10] and by the TRIM-2003 code are also shown.

Fig.2	Number of defects calculated with the help of the IOTA code and the MARLOWE code [10] for Li4SiO4 irradiated with the 28Si. Effective threshold energy is the same as for Table 2 data.

Table 2

Number of defects calculated with the help of the different codes and by the NRT formula. Effective threshold energy is equal to 17 eV for Be, 33 eV for Li, 19 eV for O and 112 eV for Si (taken for the purpose of comparison only).

| Code | 9Be in 9Be EBe=6 MeV | 4He in 9Be EHe=13 MeV | 28Si in Li4SiO4 ESi=2.3 MeV |
| --- | --- | --- | --- |
| IOTA (Integration,Eq.(5)) | 481 | 165 | 2530 |
| IOTA (Monte Carlo) | 507 | 180 | 2360 |
| MARLOWE [10] | 580 | 140 | 2500 |
| TRIM -2003 | 577 | 217 | 2200 |
| NRT formula | 436 | 96 | 2780 |

REFERENCES

- J.Lindhard, M.Scharff, H.E.Schiott, Range Concepts and Heavy Ion Ranges, K. Dan. Vidensk. Selsk. Mat. Fys. Medd., 33, N14 (1963)

- K.B.Winterbon, P.Sigmund, J.B.K.Sanders, Spatial Distribution of Energy Deposited by Atomic Particles in Elastic Collisions, K. Dan. Vidensk. Selsk. Mat. Fys. Medd., 37, N14 (1970)

- A.F.Burenkov, F.F.Komarov, M.A.Kumahov, M.M.Temkin, Spacial Distribution of Energy Deposited in Atomic Collision Cascades in Matter, Energoatomizdat, Moscow, 1985

- M.T.Robinson, Basic Physics of Radiation Damage Production, J. Nucl. Mat. 216 (1994) 1-28

- L.R.Greenwood, Neutron Interactions and Atomic Recoil Spectra, , J. Nucl. Mat. 216 (1994) 29-44

- J.P.Biersack, L.G.Haggmark, A Monte Carlo Computer Program for the Transport of Energetic Ions in Amorphous Targets, Nucl. Instr. Meth. 174 (1980) 257-269

- J.F.Ziegler, Particle Interactions with Matter, http://www.srim.org

- M.J.Norgett, M.T.Robinson, I.M.Torrens, A Proposed Method of Calculating Displacement Dose Rates, Nuclear Engineering and Design, 33 (1975) 50-54.

- GNU Fortran G77 for Windows 95/98/Me/NT4/2000/XP, http://kkourakis.tripod.com

- D.Leichte, Displacement Damage Parameters for Fusion Breeder Blanket Materials Based on BCA Computer Simulations J. Nucl. Mat. 307-311 (2002) 793-797

- A.Yu.Konobeyev, Yu.A.Korovin, V.N.Sosnin, Neutron Displacement Cross-Sections for Structural Materials below 800 MeV J. Nucl. Mat. 186 (1992) 117

- Yu.A.Korovin, A.Yu.Stankovsky, A.Yu.Konobeyev, P.E.Pereslavtsev BISERM-2. Nuclear Data Library for Evaluation of Radiation Effects in Materials Induced by Neutrons of Intermediate Energies, IAEA-NDS-203 (March 1997).

- W.Eckstein, Computer Simulation of Ion-Solid Interactions, Springer-Verlag, 1991, p.59.

- U.Littmark, J.F.Ziegler, Handbook of Range Distributions, Pergamon Press, 1985.

- T.W.Armstrong, K.C.Chandler, Nucl. Instr. Meth. 113 (1973) 313

- H.G.Hughes, H.W.Egdorf, F.C.Gallmeier, J.S.Hendricks et al: MCNPXTM User’s manual, Version 2.4.0, September 2002, LA-CP-02-408

Description of the diskette with the IOTA code distributed with this report. Windows version using GNU FORTRAN Compiler

The diskette contains the following information

Directory	Content

DESCRIPTION	this report

FORTRAN	the source of the IOTA and the SL_LNS4A codes

EXE	executable modules of the IOTA code and the SL_LNS4A code (Windows)

EXAMPLE	example of the input data

GNU_FORTRAN_G77	reference with instruction about GNU FORTRAN installation

To perform the test calculation

- create the working directory on PC hard disk (e.g. C:\). Call it, e.g. IOTACODE

- copy all executable files from the diskette\EXE directory to the C:\IOTACODE

- copy the file INPUT and all files with *.dat and *.zie extensions from diskette\EXAMPLE to C:\IOTACODE

- run  iota.exe  and compare the output files with the files located in diskette\EXAMPLE directory

To check SL_LNS4A code execution run SL_LNS4A.exe and compare the files created with the corresponding files from diskette\EXAMPLE

To check data for the stopping power install the SRIM/TRIM code from www.srim.org, prepare the necessary data and compare them with the data with *.zie extension from the diskette\EXAMPLE directory.

To install GNU FORTRAN Compiler go in directory A:\GNU_FORTRAN_G77 and open file gnu.html by the Internet browser. Follow the instruction and install “A minimal set of G77”.

The text of the IOTA code

**************************************************************************

*

* IOTA code

* ---------

*

*

* Maximal number of the material component is 21. To increase this

* value change parameter(maxc= to N desirable value in all routines

* and maxc1= to N+1

*

* Maximal energy = 5 GeV

*

*

c

c GNU random number generator is used now. To use other generator

c change function RANDOM

c

*

* Last changes 09/01/2004

*

c

PROGRAM IOTA

parameter (maxe=50001)

Common/EIN/EIN(MAXE),NH(MAXE),KEIN

COMMON/RECSTORE/TKIN(50003),ZZ(50003),NKO(50003),INDIC(50003),

#                 NREC

Common/MERCURY/CMD(5),Ecrit,MD

c

c======================================================================

c

c

*

* read input data file

CALL READ_INPUT

*

* prepare basic data for calculations

CALL PRECALCULATION

*

* read cross-sections and dE/dx

CALL READ_DATA

*

* make interpolation between data points

CALL INCREASE_DATA_ARRAY

*

* calculate dE/dx by SPAR routine

CALL DEDX_SPAR

*

*

c

c========================================================================

c

* Cycle for energies

* ------------------

Do IE=1,KEIN

T0=EIN(IE)

NHIST=NH(IE)

c

If(T0.gt.5000.) then

Print *,' T0 exceeds the allowable maximum. Change the code'

Call Error('    ',5000)

Endif

*

Call INITIALIZE_AND_ZEROIZE(T0)

*

* Cycle for events

* ----------------

Do 7 IH=1,Nhist

Print 1000,IH,T0

1000   Format('*************  history =',I10,'     E=',g12.5)

*

Call DISPERS(1)

*

* Primary ion movement

*                    IDE corresponds to the energy of the primary

*                        particle which attenuates in material

Z=0.0

IK=1

IDE=-1

Call MODEL(IK,T0,Z,IDE)

if(NREC.le.0) goto 6

*

* Secondary recoils movement

2    Call STORE_OUT(MK,T1,Z,IDE)

*

if(nrec.lt.0.and.md.eq.0) then

print *,'strange error'

stop

endif

if(NREC.le.0) goto 6

IK=MK+1

Call MODEL(IK,T1,Z,IDE)

goto 2

*

*

*

6    Call DISPERS(2)

7    Continue

c

*

* Integration of nu(T)dS/dT along the trajectory

Call  LindInt(T0)

Print *,'*****  Finished !'

*

Call Print(T0,Nhist)

c

Enddo !   Do IE=1,KEIN

*

Call end_of_calculation

Stop

End

*

************************************************************************

*

Subroutine ARRAYINT(KEY, E,CS,N,T,INTS0,RES)

*       --------------------------------------------

* REAL*4

*

*  To get RES value for arrays E(N),CS(N) at point T depending

*  ----------------------------------------------------------

*  from INTS int scheme

*  -------------------

*

* KEY defines how to treat situation when T is not covered by E array

* ---

* KEY = 0   :  stop when T < E(1)  or T > E(N)

*     = 1   :  assign RES=0.0 if T < E(1) and RES=CS(N) if T > E(N)

*

* INT has the usual meaning as in ENDF/B format

*

* Attention (!)

* ---------

* Input arrays E(N) and CS(N) should be in increased order

*

*

Dimension E(N), CS(N)

if(KEY.lt.0.or.KEY.gt.1) then

Print *,'                        E R R O R         '

Print *,'Subroutine  ARRAYINT:  KEY=',KEY,' is not acceptable !'

print *,'                            press any key...'

pause

stop

endif

INTS=INTS0

*

If(T.lt.E(1)) goto 8000

If(T.gt.E(N)) goto 9000

*

Do i=1,N

i1=i

If( T.le.E(i) ) goto 1000

Enddo

goto 9500

1000   if(i1.eq.1) then  ! means that T=E(1)

RES=CS(1)

Return

Endif

X1=E(i1-1)

X2=E(i1)

Y1=CS(i1-1)

Y2=CS(i1)

WL=T

CALL INTERMEDIATE(X1,X2,Y1,Y2,WL,INTS,RESINT)

RES=RESINT

Return

*

*

* T < E(1)

* --------

8000   if(key.eq.0) then

Write(6,*)'                        E R R O R        '

Write(6,8001) T,E(1),key

8001   Format(1x,'Subroutine  ARRAYINT:'/1x,

# ' Requested energy T=',e12.5,' is less than minimal array '

# ,'E(1) value=',e12.5

#  //1x,'              CHANGE THE DATA or KEY number !!'/1x,

# ' now  KEY is equal to', i5)

print *,'                            press any key...'

pause

stop

endif

c

RES=0.0

Print 8002,T,RES

8002   Format(1x,'ARRAYINT(N): for T =',e12.5,' result ',

# 'is set to',e12.5,' (min)')

Return

*

* T > E(N)

* --------

9000   if(key.eq.0) then

Write(6,*)'                        E R R O R        '

Write(6,9001) T,E(N),key

9001   Format(1x,'Subroutine  ARRAYINT:'/1x,

# ' Requested energy T=',e12.5,' is more than MAXIMAL array '

# ,'E(N) value=',e12.5

#  //1x,'              CHANGE THE DATA or KEY number !!'/1x,

# ' now  KEY is equal to', i5)

print *,'                            press any key...'

pause

stop

endif

c

RES=CS(N)

Print 9002,T,RES

9002   Format(1x,'ARRAYINT(X): for T =',e12.5,' result ',

# 'is set to',e12.5,' (MAX)')

Return

*

9500   Write(6,9501)

9501   Format(1x,'Subroutine  ARRAYINT:  Strange E R R O R')

print *,'                            press any key...'

pause

stop

End

*

************************************************************************

*

Function CF(IK,MK,TREC)

*      -----------------------

parameter (maxc=21,maxc1=22)

COMMON/NRT1/ANRT(MAXC1),BNRT(MAXC1),GNRT(MAXC1)

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

EDkeV=ED(MK)*1000.

* NRT

TT=TREC*1000.

*

If(TT.lt.EDkeV) then

CF=0.0

Return

Endif

If(TT.lt.2.*EDkeV) TT=2.*EDkeV

*

CF=(0.8/(2.*EDkeV))*TT/

# (1.+ANRT(IK)*TT+BNRT(IK)*(TT**0.75)+GNRT(IK)*(TT**0.16666666666))

RETURN

END

*

************************************************************************

*

FUNCTION CF1(ED,ATRN,BTRN,GTRN,TREC)

*      ------------------------------------

* As CF, but for special purpose

TT=TREC*1000.

CF1=(0.8/(2.*ED*1000.))*TT/

# (1.+ATRN*TT+BTRN*(TT**0.75)+GTRN*(TT**0.16666666666))

RETURN

END

*

************************************************************************

*

* Special function to incorporate MD result

*

Function CF2(IK,MK,TREC)

*      ------------------------

parameter (maxc=21,maxc1=22)

COMMON/NRT1/ANRT(MAXC1),BNRT(MAXC1),GNRT(MAXC1)

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/MERCURY/CMD(5),Ecrit,MD

EDkeV=ED(MK)*1000.

* mod NRT

TT=TREC*1000.   ! MeV --> keV

*

If(TT.lt.EDkeV) then

CF2=0.0

Return

Endif

*

If(TT.lt.2.*EDkeV) TT=2.*EDkeV

*

*

eMD= TT/

# (1.+ANRT(IK)*TT+BNRT(IK)*(TT**0.75)+GNRT(IK)*(TT**0.16666666666))

*

* usual formula

CF2=(0.8/(2.*EDkeV))*eMD

*

Elow =CMD(4)

Ehigh=CMD(5)

*

*

if(eMD.lt.Elow)  eMD=Elow

if(eMD.gt.Ehigh) eMD=Ehigh

efficiency=CMD(1)*(  eMD**CMD(2)  ) + CMD(3)*eMD

*

*

* modified NRT

CF2=efficiency*CF2

*

RETURN

END

*

************************************************************************

*

Subroutine COS_DEFINE(IK,MK,T0,TPVA,COST)

*       ----------------------------------------

parameter (maxc=21,maxc1=22)

*

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/PKA/ALPHA2(MAXC1,MAXC),E0(MAXC1,MAXC)

*

* T(M2)=[ 2M1*M2/(M1+M2)**2 ]* (  1-COS( TETA(CM) )) * T

*

* TG(TETA(LC))=SIN(TETA(CM))/( M1/M2 + COS(TETA(CM)) )

*

T=T0

TM2=TPVA

* cos(teta) in CM

COSTCM=1.-TM2/(T*0.5*ALPHA2(IK,MK))

if(COSTCM.gt.1.) COSTCM =0.99999999

if(COSTCM.lt.-1.) COSTCM=-0.9999999

SINTCM=Sqrt(1.-COSTCM**2)

A11=A1

If(IK.ne.1) A11=AT(IK-1)

*

TETA=ATAN(   SINTCM/( (A11/AT(MK))+COSTCM )   )

COST=COS(TETA)

c       write(445,*)'COS(TCM),COST=',COSTCM,COST

*

RETURN

END

*

************************************************************************

*

Subroutine CRITICAL_ENERGY

*       --------------------------

parameter (maxc=21,maxc1=22)

*

* search root Tdam(E)-Ecrit=0, it is supposed, that d^2Tdam/dE^2 < 0

*

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

COMMON/NRT1/ANRT(MAXC1),BNRT(MAXC1),GNRT(MAXC1)

Common/MERCURY/CMD(5),Ecrit,MD

Common/SATURN/TcritLAB(MAXC1)

*

* Tcrit correspods to  Abs( Tdam(E)-Ecrit ) < Err1

Err1=1.E-4

*

NK1=NK+1

If(Ecrit.le.0.0) Call Error('Ecri',0)

*

Do 70000 IK=1,NK1

*

X0=Ecrit

If(Tdam(IK,X0)-Ecrit)1,8000,90000

*

1    h=0.1

2    XU=X0*(1.+h)

RK=(Tdam(IK,XU)-Tdam(IK,X0))/(XU-X0)

X1=X0-(Tdam(IK,X0)-Ecrit)/RK

* Tcrit > 0.1 GeV ?

If(X1.ge.1.e+5) then

Tcrit=X1

Print 91000,Tcrit

goto 9000

endif

If(abs(Tdam(IK,X1)-Ecrit).lt.err1) then

Tcrit=X1

goto 9000

endif

If(Tdam(IK,X1)-Ecrit)1000, 8500, 2000

1000    h=0.1

X0=X1

goto 2

2000    h=h/2.

goto 2

c

8000    Tcrit=X0

goto 9000

8500    Tcrit=X1

goto 9000

c

9000    continue

*

* keV --> MeV

TcritLAB(ik)=Tcrit*0.001

70000   continue

*

*

Return

*

*

90000   Print *,'Tcrit search failed !  Tdam(Xinitial) must be < Ecrit'

Print *,'                                  press any key...'

pause

stop

91000   Format(70('*')/'*         Energy Tcrit =',g12.5,' keV',

#  ' is not realistic',T69,'*'/70('*'))

End

*

************************************************************************

*

Subroutine DATATAB

*       ------------------

c A, W (amu), Ro (g/cm3) , <I> (eV) for elements

c

c Data from SRIM code

c  <I> is mean ionization potential (eV) checked by data in figures

c  presented in http://www.srim.org/SRIM/SRIMPICS/IONIZ.htm,

c  except data noted below

c

c subroutine called from PRECALCULATION

c

Common/DATATAB1/WTAB1(101),ROTAB1(101),RITAB1(101),IATAB1(101)

Dimension IATAB(92), WTAB(92), ROTAB(92), RITAB(92)

*

*  H

Data IATAB( 1),  WTAB( 1),  ROTAB( 1),  RITAB( 1)

& /        1   ,    1.0079,   0.071486,    19.60/

* He

Data IATAB( 2),  WTAB( 2),  ROTAB( 2),  RITAB( 2)

& /        4   ,    4.0026,   0.125880,    39.00/

* Li

Data IATAB( 3),  WTAB( 3),  ROTAB( 3),  RITAB( 3)

& /        7   ,    6.9410,   0.534000,    42.20/

* Be

Data IATAB( 4),  WTAB( 4),  ROTAB( 4),  RITAB( 4)

& /        9   ,    9.0122,   1.848000,    63.70/

*  B

Data IATAB( 5),  WTAB( 5),  ROTAB( 5),  RITAB( 5)

& /       11   ,   10.8110,   2.350200,    76.00/

*  C

Data IATAB( 6),  WTAB( 6),  ROTAB( 6),  RITAB( 6)

& /       12   ,   12.0110,   2.253000,    79.10/

*  N

Data IATAB( 7),  WTAB( 7),  ROTAB( 7),  RITAB( 7)

& /       14   ,   14.0070,   1.026000,    84.20/

*  O

Data IATAB( 8),  WTAB( 8),  ROTAB( 8),  RITAB( 8)

& /       16   ,   15.9990,   1.426000,    96.00/

*  F

Data IATAB( 9),  WTAB( 9),  ROTAB( 9),  RITAB( 9)

& /       19   ,   18.9980,   1.111100,   107.00/

* Ne

Data IATAB(10),  WTAB(10),  ROTAB(10),  RITAB(10)

& /       20   ,   20.1797,   1.204000,   133.00/

* Na

Data IATAB(11),  WTAB(11),  ROTAB(11),  RITAB(11)

& /       23   ,   22.9898,   0.970000,   149.00/

* Mg

Data IATAB(12),  WTAB(12),  ROTAB(12),  RITAB(12)

& /       24   ,   24.3050,   1.736600,   156.00/

* Al

Data IATAB(13),  WTAB(13),  ROTAB(13),  RITAB(13)

& /       27   ,   26.9815,   2.702000,   170.00/

* Si

Data IATAB(14),  WTAB(14),  ROTAB(14),  RITAB(14)

& /       28   ,   28.0860,   2.321200,   171.00/

*  P

Data IATAB(15),  WTAB(15),  ROTAB(15),  RITAB(15)

& /       31   ,   30.9737,   1.821900,   173.00/

*  S

Data IATAB(16),  WTAB(16),  ROTAB(16),  RITAB(16)

& /       32   ,   32.0660,   2.068600,   184.00/

* Cl

Data IATAB(17),  WTAB(17),  ROTAB(17),  RITAB(17)

& /       35   ,   35.4530,   1.895600,   178.00/

* Ar

Data IATAB(18),  WTAB(18),  ROTAB(18),  RITAB(18)

& /       40   ,   39.9480,   1.650400,   190.00/

*  K

Data IATAB(19),  WTAB(19),  ROTAB(19),  RITAB(19)

& /       39   ,   39.0983,   0.863180,   190.00/

* Ca

Data IATAB(20),  WTAB(20),  ROTAB(20),  RITAB(20)

& /       40   ,   40.0800,   1.540000,   198.00/

* Sc

Data IATAB(21),  WTAB(21),  ROTAB(21),  RITAB(21)

& /       45   ,   44.9560,   2.989000,   225.00/

* Ti

Data IATAB(22),  WTAB(22),  ROTAB(22),  RITAB(22)

& /       48   ,   47.9000,   4.518900,   231.00/

*  V

Data IATAB(23),  WTAB(23),  ROTAB(23),  RITAB(23)

& /       51   ,   50.9420,   5.960000,   249.00/

* Cr

Data IATAB(24),  WTAB(24),  ROTAB(24),  RITAB(24)

& /       52   ,   51.9960,   7.200000,   265.00/

* Mn

Data IATAB(25),  WTAB(25),  ROTAB(25),  RITAB(25)

& /       55   ,   54.9380,   7.434100,   275.00/

* Fe

Data IATAB(26),  WTAB(26),  ROTAB(26),  RITAB(26)

& /       56   ,   55.8470,   7.865800,   286.00/

* Co

Data IATAB(27),  WTAB(27),  ROTAB(27),  RITAB(27)

& /       59   ,   58.9330,   8.900000,   306.00/

* Ni

Data IATAB(28),  WTAB(28),  ROTAB(28),  RITAB(28)

& /       58   ,   58.6900,   8.895500,   321.00/

* Cu

Data IATAB(29),  WTAB(29),  ROTAB(29),  RITAB(29)

& /       63   ,   63.5460,   8.920000,   332.00/

* Zn

Data IATAB(30),  WTAB(30),  ROTAB(30),  RITAB(30)

& /       64   ,   65.3900,   7.140000,   330.00/

* Ga

Data IATAB(31),  WTAB(31),  ROTAB(31),  RITAB(31)

& /       69   ,   69.7200,   5.904000,   334.00/

* Ge

Data IATAB(32),  WTAB(32),  ROTAB(32),  RITAB(32)

& /       74   ,   72.6100,   5.350000,   335.00/

* As

Data IATAB(33),  WTAB(33),  ROTAB(33),  RITAB(33)

& /       75   ,   74.9220,   5.727000,   353.00/

* Se

Data IATAB(34),  WTAB(34),  ROTAB(34),  RITAB(34)

& /       80   ,   78.9600,   4.810000,   351.00/

* Br

Data IATAB(35),  WTAB(35),  ROTAB(35),  RITAB(35)

& /       79   ,   79.9040,   3.199000,   317.00/

* Kr

Data IATAB(36),  WTAB(36),  ROTAB(36),  RITAB(36)

& /       84   ,   83.8000,   2.602100,   348.00/

* Rb

Data IATAB(37),  WTAB(37),  ROTAB(37),  RITAB(37)

& /       85   ,   85.4700,   1.532000,   364.00/

* Sr

Data IATAB(38),  WTAB(38),  ROTAB(38),  RITAB(38)

& /       88   ,   87.6200,   2.600000,   366.00/

*  Y

Data IATAB(39),  WTAB(39),  ROTAB(39),  RITAB(39)

& /       89   ,   88.9050,   4.469000,   375.00/

* Zr

Data IATAB(40),  WTAB(40),  ROTAB(40),  RITAB(40)

& /       90   ,   91.2200,   6.490000,   389.00/

* Nb

Data IATAB(41),  WTAB(41),  ROTAB(41),  RITAB(41)

& /       93   ,   92.9060,   8.570000,   417.00/

* Mo

Data IATAB(42),  WTAB(42),  ROTAB(42),  RITAB(42)

& /       98   ,   95.9400,  10.206000,   423.00/

* Tc

Data IATAB(43),  WTAB(43),  ROTAB(43),  RITAB(43)

& /       97   ,   97.0000,  11.500000,   428.00/

* Ru

Data IATAB(44),  WTAB(44),  ROTAB(44),  RITAB(44)

& /      102   ,  101.0700,  12.300000,   441.00/

* Rh

Data IATAB(45),  WTAB(45),  ROTAB(45),  RITAB(45)

& /      103   ,  102.9100,  12.399000,   468.00/

* Pd

Data IATAB(46),  WTAB(46),  ROTAB(46),  RITAB(46)

& /      106   ,  106.4000,  12.020000,   476.00/

* Ag

Data IATAB(47),  WTAB(47),  ROTAB(47),  RITAB(47)

& /      107   ,  107.8700,  10.473000,   488.00/

* Cd

Data IATAB(48),  WTAB(48),  ROTAB(48),  RITAB(48)

& /      114   ,  112.4000,   8.642000,   441.00/

* In

Data IATAB(49),  WTAB(49),  ROTAB(49),  RITAB(49)

& /      115   ,  114.8200,   7.300000,   481.00/

* Sn

Data IATAB(50),  WTAB(50),  ROTAB(50),  RITAB(50)

& /      120   ,  118.7100,   7.281600,   478.00/

* Sb

Data IATAB(51),  WTAB(51),  ROTAB(51),  RITAB(51)

& /      121   ,  121.7500,   6.684000,   472.00/

* Te

Data IATAB(52),  WTAB(52),  ROTAB(52),  RITAB(52)

& /      130   ,  127.6000,   6.250000,   485.00/

*  I

Data IATAB(53),  WTAB(53),  ROTAB(53),  RITAB(53)

& /      127   ,  126.9000,   4.937300,   455.00/

* Xe

Data IATAB(54),  WTAB(54),  ROTAB(54),  RITAB(54)

& /      132   ,  131.3000,   3.058900,   440.00/

* Cs

Data IATAB(55),  WTAB(55),  ROTAB(55),  RITAB(55)

& /      133   ,  132.9100,   1.878500,   488.00/

* Ba

Data IATAB(56),  WTAB(56),  ROTAB(56),  RITAB(56)

& /      138   ,  137.3270,   3.510000,   491.00/

* La

Data IATAB(57),  WTAB(57),  ROTAB(57),  RITAB(57)

& /      139   ,  138.9100,   6.173800,   472.00/

* Ce

Data IATAB(58),  WTAB(58),  ROTAB(58),  RITAB(58)

& /      140   ,  140.1200,   6.672400,   501.00/

* Pr

Data IATAB(59),  WTAB(59),  ROTAB(59),  RITAB(59)

& /      141   ,  140.9100,   6.773000,   513.00/

* Nd

Data IATAB(60),  WTAB(60),  ROTAB(60),  RITAB(60)

& /      142   ,  144.2400,   7.008000,   546.00/

* Pm

Data IATAB(61),  WTAB(61),  ROTAB(61),  RITAB(61)

& /      148   ,  148.0000,   6.475000,   560.00/

* Sm

Data IATAB(62),  WTAB(62),  ROTAB(62),  RITAB(62)

& /      152   ,  150.3600,   7.520000,   566.00/

* Eu

Data IATAB(63),  WTAB(63),  ROTAB(63),  RITAB(63)

& /      153   ,  151.9700,   5.244000,   580.00/

* Gd

Data IATAB(64),  WTAB(64),  ROTAB(64),  RITAB(64)

& /      158   ,  157.2500,   7.901000,   586.00/

* Tb

Data IATAB(65),  WTAB(65),  ROTAB(65),  RITAB(65)

& /      159   ,  158.9300,   8.230000,   614.00/

* Dy

Data IATAB(66),  WTAB(66),  ROTAB(66),  RITAB(66)

& /      164   ,  162.5000,   8.551000,   603.00/

* Ho

Data IATAB(67),  WTAB(67),  ROTAB(67),  RITAB(67)

& /      165   ,  164.9300,   8.795000,   641.00/

* Er

Data IATAB(68),  WTAB(68),  ROTAB(68),  RITAB(68)

& /      166   ,  167.2600,   9.066000,   661.00/

* Tm

Data IATAB(69),  WTAB(69),  ROTAB(69),  RITAB(69)

& /      169   ,  168.9300,   9.321000,   683.00/

* Yb

Data IATAB(70),  WTAB(70),  ROTAB(70),  RITAB(70)

& /      174   ,  173.0400,   6.960000,   684.00/

* Lu

Data IATAB(71),  WTAB(71),  ROTAB(71),  RITAB(71)

& /      175   ,  174.9700,   9.841000,   694.00/

* Hf

Data IATAB(72),  WTAB(72),  ROTAB(72),  RITAB(72)

& /      180   ,  178.4900,  13.310000,   675.00/

* Ta

Data IATAB(73),  WTAB(73),  ROTAB(73),  RITAB(73)

& /      181   ,  180.9500,  16.601000,   729.00/

*  W

Data IATAB(74),  WTAB(74),  ROTAB(74),  RITAB(74)

& /      184   ,  183.8500,  19.350000,   735.00/

* Re

Data IATAB(75),  WTAB(75),  ROTAB(75),  RITAB(75)

& /      187   ,  186.2000,  20.530001,   749.00/

* Os

Data IATAB(76),  WTAB(76),  ROTAB(76),  RITAB(76)

& /      192   ,  190.2000,  22.480000,   746.00/

* Ir

Data IATAB(77),  WTAB(77),  ROTAB(77),  RITAB(77)

& /      193   ,  192.2000,  22.421000,   757.00/

* Pt

Data IATAB(78),  WTAB(78),  ROTAB(78),  RITAB(78)

& /      195   ,  195.0800,  21.450001,   792.00/

* Au

Data IATAB(79),  WTAB(79),  ROTAB(79),  RITAB(79)

& /      197   ,  196.9700,  19.311001,   789.00/

* Hg

Data IATAB(80),  WTAB(80),  ROTAB(80),  RITAB(80)

& /      202   ,  200.5900,  13.546200,   800.00/

* Tl

Data IATAB(81),  WTAB(81),  ROTAB(81),  RITAB(81)

& /      205   ,  204.3800,  11.850000,   810.00/

* Pb

Data IATAB(82),  WTAB(82),  ROTAB(82),  RITAB(82)

& /      208   ,  207.1900,  11.343700,   787.00/

* Bi

Data IATAB(83),  WTAB(83),  ROTAB(83),  RITAB(83)

& /      209   ,  208.9800,   9.800000,   819.00/

* Po (no fig for <I>)

Data IATAB(84),  WTAB(84),  ROTAB(84),  RITAB(84)

& /      209   ,  210.0000,   9.251100,   830.00/

* At <I> is not well defined from SRIM data (see original files)

Data IATAB(85),  WTAB(85),  ROTAB(85),  RITAB(85)

& /      210   ,  210.0000,  10.000000,   756.95/

* Rn <I> is not well defined from SRIM data (see original files)

Data IATAB(86),  WTAB(86),  ROTAB(86),  RITAB(86)

& /      222   ,  222.0000,   9.910000,   736.54/

* Fr (no fig for <I>)

Data IATAB(87),  WTAB(87),  ROTAB(87),  RITAB(87)

& /      223   ,  223.0000,  10.000000,   827.00/

* Ra (no fig for <I>)

Data IATAB(88),  WTAB(88),  ROTAB(88),  RITAB(88)

& /      226   ,  226.0000,   5.022200,   826.00/

* Ac (no fig for <I>)

Data IATAB(89),  WTAB(89),  ROTAB(89),  RITAB(89)

& /      227   ,  227.0000,  10.000000,   841.00/

* Th (no fig for <I>)

Data IATAB(90),  WTAB(90),  ROTAB(90),  RITAB(90)

& /      232   ,  232.0000,  11.658000,   847.00/

* Pa (no fig for <I>)

Data IATAB(91),  WTAB(91),  ROTAB(91),  RITAB(91)

& /      231   ,  231.0000,  15.370000,   878.00/

*  U

Data IATAB(92),  WTAB(92),  ROTAB(92),  RITAB(92)

& /      238   ,  238.0300,  19.042999,   842.00/

c

c

Do i=1,92

IATAB1(i)=IATAB(i)

WTAB1(i) =WTAB(i)

ROTAB1(i)=ROTAB(i)

RITAB1(i)=RITAB(i)

enddo

Do i=93,101

IATAB1(i)=0

WTAB1(i) =0.0

ROTAB1(i)=0.0

RITAB1(i)=0.0

enddo

Return

End

*

************************************************************************

*

Subroutine dEdXNUC_RECALC(Z1,Z2,A1,A2,E,sN)

*      -------------------------------------------

c Z1,A1: projectile, Z2,A2: target

c

c

Real*8 Z,EL,RL,rksi,eps,t,sNd

c

c E (MeV)

c

Z=(Z1**0.666666d0+Z2**0.6666666d0)**1.5d0

c

EL=3.07358d-05*Z1*Z2*(Z**0.33333333d0)*( (A1/A2)+1.d0 )

c (no coeff, n0)

RL=(Z**0.6666666d0) *(  (A1+A2)**2  )/(A1*A2)

c

rksi=1.309d0

c

eps=E/EL

t=((2.d0*rksi)**0.6666666d0)*(eps**0.8888888d0)

c

sNd=(EL/RL)*(1.d0/eps)*

# (  -dsqrt(t)/dsqrt(1.+t)   +dlog(dsqrt(t)+dsqrt(1.d0+t))  )

c

sN=real(sNd)

Return

End

*

************************************************************************

*

Subroutine DEDX_SPAR

*      --------------------

* Calculation of dE/dx by SPAR (Armstrong, Chandler) routine

*

* This subroutine should be executed after

*     subr INCREASE_DATA_ARRAY, where NMAX is calculated

*

* units for stopping power [MeV/cm], range [cm]

*

parameter (maxc=21,maxc1=22)

Character OUTSPAR*12, C5*5

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

*

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

*

Common/STARS1/ dSdX1(MAXC1,MAXC,10002),

# dEdXNuc1(MAXC1,MAXC,10002),

# dEdXNuc0Ed1(MAXC1,MAXC,10002), dEdXNucEdTm1(MAXC1,MAXC,10002),

# CST1(MAXC1,MAXC,10002), NMAX

Common/ISTOP/ISTOP

Common/VENUS/RI(MAXC)

Dimension TmpE(10002),dxdE(10002)

*

if(istop.ne.1) return

*

*

* Condensed matter supposed

If(RO.le.0.01)  goto 91000

*

* Mean <I> for compound (only for printing)

RIL=0.d0

Do i=1,NK

RIL= RIL+FR(i)*ZT(i)*alog( RI(i) )

Enddo

RIL=2.718281828**(RIL/Z2)

*

jj=2 ! condensed element

if(NK.ne.1) then

* check if all components are isotopes of the same element

IZT0=ifix( ZT(1)+0.001)

do i=1,NK

IZT=ifix( ZT(i)+0.001)

if(IZT.ne.IZT0) then

jj=4   ! condensed compound

goto 1

endif

IZT0=IZT

enddo

endif

1     continue

*

*

NK1=NK+1

*

*

*

Do  IK=1,NK1

*     ------------

* IK - id of the moving ion,   IK=1: primary particle

*

*

* Projectile

if(IK.eq.1) then

ZPR=Z1

APR=A1

else

ZPR=ZT(IK-1)

APR=AT(IK-1)

endif

*

IZA=1000*ifix(ZPR+0.001)+ifix(APR+0.001)

Write(C5,'(i5)')IZA

outspar=C5//'.sto'

open(21,file=outspar)

Write(21,90001)ZPR,APR,IK,Z2,A2,JJ,RO,RN0,RIL*1.e+6

Write(21,90002)

Write(21,90003)

*

*

Do I=1,999999999

*

* J=(lg(E)+6)*1000+1  means that energy can changes from 10**-6 MeV

* If 10002 array dimension will be increased change also 1000 below

* and in other subroutines (i.e. MODEL)

RLG=Float(I-1)/1000. -6.

E=10.**RLG   ! Energy (MeV)

! NMAX is already defined in subr INCREASE_DATA_ARRAY and check < 5 GeV is done

If(I.gt.NMAX) goto 7000

! Attention. NMAX is the same for all material components, it is

* supposed that SL_NLS code which is executed before IOTA code

* was stopped at the same energy for all components

*

Call SPAR0(ZPR, APR, E, dEdxTOTS, dEdxNS, JJ)

*

* total stopping power

dEdx(IK,i)  =dEdxTOTS

* nuclear stopping

dEdxN1(IK,i)=dEdxNS

*

* calculate total path from 1/ dE/dx integration (lin-lin)

TmpE(i)=E

dxdE(i)=0.0

if(dEdxTOTS.ne.0.0) dxdE(i)=1./dEdxTOTS

if(i.eq.1) then

Rp1(IK,i)   =0.0

else

Rp1(IK,i)=Rp1(IK,i-1)+(TmpE(i)-TmpE(i-1))*

#           (dxdE(i)+dxdE(i-1))/2.

endif

*

*

Enddo    !  Do I1=1,999999999

7000   continue

*

* write results

* units for printing MeV/mg/cm2, cm

c                   prc=1./(1000.*RO)

prc=1.

Do i=1,NMAX

Write(21,90004) TmpE(i), prc*(dEdx(IK,i)-dEdxN1(IK,i)) ,

#                            prc*dEdxN1(IK,i), Rp1(IK,i)

Enddo

close(21)

*

*

Enddo    !  Do IK=1,NK1

*       -----------------------

*

*

iiii=1

c        if(iiii.eq.1) stop

Return

c

c

90001   Format(' Stopping power from SPAR (MCNPX)'/

#  ' projectile ',f6.1,f8.2,'       IK =',i3,

#  /' medium     ',f6.1,f8.2,'    condensed   JJ =',i2,

#  /' density =',f8.3,' g/cm3     n0 =',1pe12.5,' cm-3'

#  /' mean ioniz pot = ',0pf8.2,' eV')

90002   Format(/' units  E [MeV],  dE/dx [MeV/mg/cm2],  R [cm] ')

90003   Format(/'     E        dE/dx(electr)  dE/dx(nucl)',

#  '       R (total) '/ 61('-'))

90004   Format(1pe12.5,2e14.5,e17.5)

c

91000   Print 91001

91001   Format(/' Gaseous target is not allowed ! '

#  /20x,'Change subr DEDXSPAR  !!')

print *,'                        press any key...'

pause

stop

c

End

*

************************************************************************

*

Subroutine  DEDX_ZIEGLER_READ(IK, IR, RO0, Ex, dEdx, Rp, NdE,

#   dEdxN)

*        -------------------------------------------------------------

c

c Ex,  dEdx  - energy and total stopping power   (Ziegler/SRIM)

c      dEdxN - nuclear stopping power            (Ziegler/SRIM)

c

parameter (maxc=21)

Character C12*12,C33*33,C7*7,CID3*3,CID2*2, C17*17

Character C6*6, C80*80

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/Basic/Z1,A1,Z2,A2,AW,ROOO,RN0

Dimension Rp(1501), Ex(1501), dEdx(1501)

Dimension dEdxN(1501)

Dimension ExTMP(1501),dEdxETMP(1501), dEdxE(1501)

Dimension ExTMP2(1501),dEdxETMP2(1501)

If(IK.eq.1) then

Zini=Z1

Aini=A1

else

Zini=ZT(IK-1)

Aini=AT(IK-1)

endif

IZT=Ifix(Zini+0.001)

IAT=Ifix(Aini+0.001)

*

* find "Ion ="

*

Do i=1,111111111

read(IR,'(a6)',end=90001)C6

If(C6.eq.' Ion =')             then

backspace ir

read(ir,'(a80)')C80

*

* Get Z,A from SRIM output

Call ZieION(C80,Zzie,Azie)

*

*

goto 11

endif ! If(C6.eq.' Ion =')

Enddo

*

*

* find "Target density"

*

11      Do i=1,111111111

read(IR,'(a17)',end=90005)C17

If(C17.eq.' Target Density =') then  ! 1)

backspace ir

read(ir,'(t18,e12.0)')RO

if(RO.ne.RO0) then                ! 2)

print 1,IZT,IAT,RO0,RO

1         format(1x,'  W A R N I N G !'/1x,

#      'for ',i3,i4,' RO(input)=',g12.5,' BUT  Ziegler(RO)=',

#      g12.5/1x,'The last one will be used to recover dEdx')

print *,'                   press any key to continue'

pause

endif                ! 2)

goto 2

endif  ! 1)

Enddo

*

* find line "Stopping units"

*

2      do i=1,111111111

read(IR,'(a17)',end=90006)C17

If(C17.eq.' Stopping Units =') then

backspace ir

read(IR,'(a33)',end=90006)C33

if(C33.ne.' Stopping Units =  MeV / (mg/cm2)') then

Print *,' Units of stopping power must be MeV/mg/cm2 !'

print *,'                         press any key...'

pause

stop

endif

goto 3

endif

enddo

*

* skip 4 lines

3      do i=1,4

read(IR,*,end=90007)

enddo

*

read(IR,'(a12)')C12

if(C12.ne.'----------- ') Call error('----',12)

*

* Begin reading

*

* transformation of SRIM data

*

i=1

Ex(1)=0.0

dEdx(1)=0.0

Rp(1)=0.0

dEdxN(1)=0.0

c

c

Do iii=1,1111111       ! ---c y c l e begins----

read(IR,'(a7)')C7

if(C7.eq.'-------') goto 2000

backspace IR

read(IR,'(e7.0,1x, a3,   e12.0,   e11.0,    e8.0,1x, a2)')

#             EN,       CID3, dEdx_el, dEdx_nuc, RRR,       CID2

if(CID3.ne.'eV '.and.CID3.ne.'keV'.and.CID3.ne.'MeV'.

#   and.CID3.ne.'GeV') call ERROR(' ev?',11111)

*

* energy ---> MeV

ENERGY_MULT=1.0

if(CID3.eq.'eV ')  ENERGY_MULT=1.E-06

if(CID3.eq.'keV')  ENERGY_MULT=1.E-03

if(CID3.eq.'GeV')  ENERGY_MULT=1.E+03

*

if(CID2.ne.'A '.and.CID2.ne.'um'.and.CID2.ne.'mm'

#   .and.CID2.ne.'cm'.and.CID2.ne.'m ') call ERROR(' um?',11111)

*

* all units ---> cm

RANGE_MULT=1.

if(CID2.eq.'A ')  RANGE_MULT=1.E-08     ! 1 A=10**-10 m

if(CID2.eq.'um')  RANGE_MULT=1.E-04     ! 1 um=micron= 10**-6 m

if(CID2.eq.'mm')  RANGE_MULT=0.1        ! 1 mm=0.001 m

if(CID2.eq.'m ')  RANGE_MULT=100.       ! 1 m =100 cm

* 1)

* MeV/(mg/cm2) ----> MeV/(g/cm2)

dEdx_sum= 1000.*( dEdx_el+dEdx_nuc)

dEdx_nuc2= 1000.*dEdx_nuc

* 2)

* MeV/(g/cm2) ----> MeV/cm

dEdx_sum= dEdx_sum*RO

dEdx_nuc2= dEdx_nuc2*RO

*

Energy=EN* ENERGY_MULT

Range= RRR* RANGE_MULT

*

* fill arrays

i=i+1

if(i.gt.1501)  Call Error('DEDX',1501)

Ex(i)=Energy

dEdx(i)=dEdx_sum

dEdxN(i)= dEdx_nuc2

Rp(i)=Range

Enddo                  ! ---c y c l e finished----

call error('_Zie',11111)

*

* end of reading

2000     NdE=i

*

*

* Recalculate stopping if Z=Z', but A.ne.A'

*

IZzie=Ifix(Zzie+0.001)

If(IZzie.ne.IZT) then

Print 3000,Zini,Aini,Zzie,Azie

3000     Format(' SRIM data failure !!!'/1x,' Current incident ion is ',

#   f5.1,f8.2/20x,'BUT'/' SRIM data are prepared for ',f5.1,f8.2)

print *,'                           press any key...'

pause

stop

endif

* no recalculation if A-A' < 1

If( abs(Azie-Aini).lt.1.0) Return

*

If(NK.ne.1) then

Print 3100

3100     Format(70('*')/'*',25x,'W A R N I N G !',t70,'*'/'*',

#   4x,' for compounds stopping power will not be recalculated',

#   t70,'*'/70('*')//1x,

#   'press any key to continue  calculations ...')

pause

Return

endif

*

* Recalculation

*

* i) electronic stopping

*

Do i=1,NdE

ExTMP(i)=Ex(i)*Aini/Azie

dEdxETMP(i)=dEdx(i)-dEdxN(i)

Enddo

c

if(Aini-Azie) 4000,4000,5000

c

c A' < A ==> lack of high energy data, get them by interp two last points

c

4000        Do i=1,NdE

m=i

if(ExTMP(NdE).le.Ex(i)) goto 4010

Enddo

call Error('4010',NdE)

4010         X1=ExTMP(NdE-1)

X2=ExTMP(NdE)

Y1=dEdxETMP(NdE-1)

Y2=dEdxETMP(NdE)

INTS=2

j=NdE

Do i=m,NdE

j=j+1

if(j.gt.1501)  Call Error('j :1',1501)

ExTMP(j)=Ex(i)

WL=ExTMP(j)

Call INTERMEDIATE(X1,X2,Y1,Y2,WL,INTS,RESINT)

dEdxETMP(j)=RESINT

Enddo

NdETMP=j

goto 7000

c

c A' > A ==> lack of low energy data, get them by interp two first points

c

5000        Do i=2,NdE

m=i-1

if(ExTMP(2).le.Ex(i)) goto 5010  ! "2" is first non-zero value

Enddo

call Error('5010',NdE)

5010         X1=ExTMP(2)

X2=ExTMP(3)

Y1=dEdxETMP(2)

Y2=dEdxETMP(3)

INTS=2

j=1

ExTMP2(j)=0.0

dEdxETMP2(j)=0.0

c

Do i=2,m

j=j+1

ExTMP2(j)=Ex(i)

WL=ExTMP2(j)

Call INTERMEDIATE(X1,X2,Y1,Y2,WL,INTS,RESINT)

dEdxETMP2(j)=RESINT

Enddo

c

Do i=2,NdE

j=j+1

if(j.gt.1501)  Call Error('j :2',1501)

ExTMP2(j)=ExTMP(i)

dEdxETMP2(j)=dEdxETMP(i)

Enddo

NdETMP=j

Do i=1,NdETMP

ExTMP(i)=ExTMP2(i)

dEdxETMP(i)=dEdxETMP2(i)

Enddo

c

c recover old energy grid

c

7000     Do i=1,NdE

E=Ex(i)

Call ARRAYINT(0,ExTMP,dEdxETMP, NdETMP, E, 2, RES)

dEdxE(i)=RES

Enddo

*

* ii) nuclear stopping

*

Do i=2,NdE

E=Ex(i)

Call dEdXnuc_recalc(Zzie,Z2,Azie,A2,E,sNzie)

Call dEdXnuc_recalc(Zini,Z2,Aini,A2,E,sNini)

If(sNzie.eq.0.0) Call error('snzi',0)

c

dEdxN(i)=dEdxN(i)*sNini/sNzie

Enddo

*

* total stopping

open(22,file='renorm.sto') ! test printing

Write(22,7500)Zini,Aini,Zzie,Azie,Z2,A2

7500   Format(' Incident ion  ', f5.1,f8.2/

# ' SRIM data for ',f5.1,f8.2/25x,' Target ',f5.1,f8.2//

#' Energy [MeV],   Stopping power [MeV/cm]'//

#'     E         dE/dx(elec)     dE/dx(nucl) '/44('-'))

c

Do i=1,NdE

dEdx(i)=dEdxN(i)+dEdxE(i)

Write(22,'(1pe11.4,e14.3,e16.3)')Ex(i),dEdxE(i),dEdxN(i)

Enddo

c

Return

c

c

90001    Print *,'------- End of SRIM output file ------'

Print *,'  Phrase "Ion =" is not found'

print *,'                         press any key...'

pause

stop

90005    Print *,'-------  End of SRIM output file ------'

Print *,'  Phrase "Target density" is not found'

print *,'                         press any key...'

pause

stop

90006    Print *,'-------  End of SRIM output file ------'

Print *,'  Phrase "Stopping units" is not found'

print *,'                         press any key...'

pause

stop

90007    Print *,'-------  End of SRIM output file ------'

Print *,'               UNEXPECTED !'

print *,'                         press any key...'

pause

stop

End

*

************************************************************************

*

Subroutine DISPERS(KEY)

*      -----------------------

Common/SUNDI/DIVel1(405),DIFRENKEL(405),DIVel5

Common/SUN/Vel1L(405),Tmean1,Vel2Tm(405),Vel3Pl(405),

#            R1,R1p,Tplay1,Vel4(405),Nel(405)

Common/MEMORY/NFRENKEL(405)

Common/SUN2/ZV5,Vel5,NN(405)

*

Common/DIMEMO/DV1(405),DFREN(405),DV5

*

If(Key.eq.2) goto 2

* store data before the history

Do i=1,405

DV1(i)=Vel1L(i)

DFREN(i)=float(NFRENKEL(i))

Enddo

DV5=Vel5

Return

* sum

2     continue

SUMV1=0.0

SUMF=0.0

Do i=1,405

SUMV1=SUMV1+Vel1L(i)-DV1(i)   ! = result for the current history

SUMF= SUMF +float(NFRENKEL(i))-DFREN(i)

DIVel1(i)    =DIVel1(i)   +SUMV1**2

DIFRENKEL(i) =DIFRENKEL(i)+SUMF**2

Enddo

*

DIVel5=DIVel5+ (Vel5-DV5)**2

Return

End

*

************************************************************************

*

Subroutine END_OF_CALCULATION

*       -----------------------------

Write(33,1)

1     Format(33(1h*),'e*n*d',35(1h*))

Return

end

*

************************************************************************

*

Subroutine Error(phrase,I)

*        --------------------------

Character phrase*4

write(6,1)phrase,I

1        Format(1x,'        ???   ',a4/1x,

*   'ERROR !!!          number=',i7)

print *,'                       press <Enter>...'

pause

stop

end

*

************************************************************************

*

FUNCTION F(X)

*        -------------

* Scattering f(x)= f(t^1/2) function

*

Common/PARAM01/IDSDT

Common/JUPITER/ALP1,ALP2,BET1,BET2,BET3,BET4

*

If(IDSDT.eq.0)  then

*

F=ALP1*(X**BET1)*(   ( 1.+( ALP2*(X**BET2) )**BET3)**BET4  )

Return

else

*

* Burenkov et al function

*

F=FBUR(X)

Return

endif

End

*

************************************************************************

*

FUNCTION FBUR(X)

*        ---------------

* Burenkov et al function

If(X.le.0.04) then

RJ=1.

A1=0.088

A2=1.

A3=0.2

goto 1

endif

If(0.04.lt.X  .and.  X.le.1.0) then

RJ=1.

A1=0.23

A2=1.3

A3=0.4

goto 1

endif

If(1.0.lt.X  .and.  X.le.40.) then

RJ=0.0

A1=0.4563

A2=0.0

A3=0.3

goto 1

endif

If(X.gt.40.) then

RJ=0.0

A1=0.5

A2=0.0

A3=0.0

endif

1        FBUR=A1*(X**(RJ/2.))*(X+A2)/(  (X+A3)**2 )

Return

End

*

************************************************************************

*

Function FINDP(S80,C10)

*        ---------------------------------

* Find phrase C10*XX (here XX=1) in line S80*80

* N = position of the last symbol of C10 in S80

* N = 0 if the phrase not found

*

Character S80*80, C10*1

NS=80

NC=1

N=0

Do 2000 I=1,NS

i1=I

i2=I+NC-1

if(i2.gt.NS) goto 2000

if(s80(I1:I2).eq.C10) goto 3000

2000     continue

findp=float(N)

return

*

3000     continue

N=i2

findp=float(N)

return

end

*

******************************************************************

*

Subroutine FT12CHOICE(idsdt)

*        --------------------------

* Main formula

* f(x)=Lambda* x**(1-2m) [1+ {2*Lambda*x**(2*(1-m)}**q ]**(-1/q)

*

* Parameters are taken mainly from   W.Eckstein, "Computer Simulation

* of Ion-Solid Interactions"  Springer, Berlin, 1991, p.59, Table 4.8

*

*

* alp1 = L,  bet1 = (1-2*m),  alp2 = 2*L

* bet2 = 2*(1-m), bet3= q,    bet4 = -1/q

*

Common/JUPITER/ALP1,ALP2,BET1,BET2,BET3,BET4

Common/AUX1/rlambda,rm,q

*

if(idsdt.eq.1) return   ! --> Burenkov et al function

if(idsdt.eq.0) goto 10

goto(888,888,888,888,888,888,888,888,888,10,

#        11,12,13,14,15), idsdt

goto 888

*

* Original Winterbon et al parameters

10       rlambda=1.309

rm=1./3.

q=2./3.

goto 777

*

* Thomas-Fermi-Sommerfeld

11       rlambda=1.7

rm=0.311

q=0.588

goto 777

*

* Bohr

12       rlambda=2.37

rm=0.103

q=0.570

goto 777

*

* Lenz-Jensen

13       rlambda=2.92

rm=0.191

q=0.512

goto 777

*

* Moliere

14       rlambda=3.07

rm=0.216

q=0.530

goto 777

*

* U.Littmark, J.F.Ziegler, "Handbook of Range Distributions"

* Kr-C potential

15       rlambda=3.35

rm=0.2328

q=0.4445

*

*

777      alp1=rlambda

bet1=1.-2.*rm

alp2=2.*rlambda

bet2=2.*(1.-rm)

bet3=q

bet4=-1./q

!

idsdt=0

Return

*

888    Print 999,idsdt

999    Format(' ERROR:    Parameters of f(t^1/2) are not known !'/

#   '           idsdt=',i5,' is not correct.')

Print *,'                     press any key...'

pause

stop

End

*

************************************************************************

*

Subroutine GET_FILE_NAMES(IU,NK,NF)

*       ----------------------------------

*

* To get NK-number file names (max 12 charactres) from input lines

* written in arbitrary order.

*

*    Any number of lines can be used to write NK file names

* Example (NK=7) (no "c" in real input):

c---------------e x a m p l e-----------------

c abc.dat    cde.dat

c   qweert.dat q.dat   w.w   www.dat  iuiu.inp

c---------------------------------------------

*    Any line beginning from * is the comment and will be omitted.

*    Reading stops when all NK names will be obtained.

*    Input line lenght is max 80 characters.

*

* IU     - number of input device

* NK     - number of names of the files to be read

* NF(NK) - name of files

*

Character C80*80, NF*12, NFTMP*12

Dimension NF(NK)

c

400    Read(IU,'(a80)') C80

if(C80(1:1).eq.'*') goto 400

MK=0

1      Do J=1,80

i=J

If(C80(j:j).ne.' ') goto 2

Enddo

* only blank line left

If(MK.gt.NK)  goto 90000

If(MK.eq.NK)  return

* MK < NK

500    Read(IU,'(a80)') C80

if(C80(1:1).eq.'*') goto 500

goto 1

*

2      MK=MK+1

If(MK.gt.NK)  goto 90000

NFTMP='            '

L=0

Do M=i,80

If(C80(m:m).eq.' ') goto 3

L=L+1

If(L.gt.12) goto 91000

NFTMP(L:L)=C80(M:M)

C80(M:M)=' '

Enddo

3      NF(MK)=NFTMP

If(MK.eq.NK) Return

goto 1

*

90000   Print *,'***** Subr.GET_FILE_NAMES'

Print *,' Number of files MK exceeds NK(input) !'

print *,'                         press any key...'

pause

stop

91000   Print *,'***** Subr.GET_FILE_NAMES'

Print *,' One of the file name exceeds 12 characters !'

Print *,'               CHANGE INPUT   '

print *,'                         press any key...'

pause

stop

*

END

*

************************************************************************

*

Subroutine GRAPH(Y,F,N3,IALOG)

*     ------------------------------

* IALOG .ne.0 : Log scale for Y

COMMON/MAIIPR/IPR1

DIMENSION Y(N3),F(N3),F1(11250)

INTEGER A1(40),B1,C1,D1,D2,D3,D4,D5,D6

DATA B1,C1,D1,D2,D5,D6/1H*,1H ,1H!,1H-,1HI,1H./

IF(N3.GT.11250)then

Print*,' BAD operation in the code GRAPH'

STOP

endif

N1=1

W=F(1)

DO 1 L=1,N3

IF(ABS(W).GT.ABS(F(L)))GOTO 1

W=F(L)

1 CONTINUE

IF(W.NE.0.)GOTO 70

WRITE(IPR1,41)

41 FORMAT(1X,' GRAPH:  maximal value identically equal to zero')

RETURN

70 IF(IALOG.NE.0)GOTO 199

WRITE(IPR1,60)

60 FORMAT(/3X,'   X           Y       ',4X,4('.........!'))

T=40./ABS(W)

DO 300 I=1,N3

300 F1(I)=F(I)

GOTO 7

199 WMIN=1.E+10

DO 200 I=1,N3

IF(F(I).NE.0..AND.F(I).LT.WMIN)WMIN=F(I)

200 CONTINUE

IMAX=-8

210 IF(W.LE.10.**IMAX)GOTO 220

IMAX=IMAX+1

GOTO 210

220 IMIN=IMAX

230 IF(WMIN.GE.10.**IMIN)GOTO 240

IMIN=IMIN-1

GOTO 230

240 R10=10.**IMIN

IF(IMIN.EQ.IMAX)GOTO 250

WW1=FLOAT(IMIN)

WW=FLOAT(IMAX-IMIN)

R20=10.**IMAX

WRITE(IPR1,62)R10,R20

62 FORMAT(1X,5X,'semilogarithmic net        '/3X,23X,E8.2,32X,E8.2)

GOTO 260

250 WW1=ALOG10(WMIN)

WW=ALOG10(WMAX)-ALOG10(WMIN)

260 DO 270 I=1,N3

FF=F(I)

IF(FF.EQ.0.)FF=R10

270 F1(I)=ALOG10(FF)-WW1

WRITE(IPR1,60)

T=40./WW

7 CONTINUE

DO 3 I=1,N3

X=F1(I)

N=INT(X*T)

IF(I.EQ.N3)GOTO 105

IF(I.EQ.N1*10)GOTO 107

D4=C1

GOTO 108

105 D4=D6

GOTO 108

107 D4=D2

N1=N1+1

108 N2=1

DO 4 J1=1,40

IF(J1.EQ.20)GOTO 110

IF(J1.EQ.1.OR.J1.EQ.40)GOTO 24

D3=D4

GOTO 25

24 D3=D5

25 CONTINUE

GOTO 111

110 D3=D1

N2=N2+1

111 CONTINUE

IF(J1.EQ.N)GOTO 101

A1(J1)=D3

GOTO 102

101 A1(J1)=B1

102 CONTINUE

4 CONTINUE

WRITE(IPR1,61)Y(I),F(I),A1

61 FORMAT(1PE12.5,1X,E12.5,5X,40A1)

3 CONTINUE

RETURN

END

*

************************************************************************

*

Subroutine INCREASE_DATA_ARRAY

*      ------------------------------

parameter (maxc=21,maxc1=22)

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

*

Common/dEdxMain01/Ex01(MAXC1,1501), dEdx01(MAXC1,1501),

# Rp01(MAXC1,1501), dEdxN01(MAXC1,1501),NdE01(MAXC1)

*

Common/Lindhard01/Energy01(MAXC1,MAXC,1701),

# dSdX01(MAXC1,MAXC,1701), dEdX_Nuc01(MAXC1,MAXC,1701),

# dEdX_Nuc_0Ed01(MAXC1,MAXC,1701),

# dEdX_Nuc_Ed_Tmax01(MAXC1,MAXC,1701),

# CST01(MAXC1,MAXC,1701), NSL01(MAXC1,MAXC)

*

* corresponds to /dEdxMain01/

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

* corresponds to /Lindhard01/

Common/STARS1/ dSdX1(MAXC1,MAXC,10002),

# dEdXNuc1(MAXC1,MAXC,10002),

# dEdXNuc0Ed1(MAXC1,MAXC,10002), dEdXNucEdTm1(MAXC1,MAXC,10002),

# CST1(MAXC1,MAXC,10002), NMAX

*

Common/istop/istop

Dimension TMP1(1701),TMP2(1701),TMP3(1701),TMP4(1701),

#           TMP5(1701),TMP6(1701)

*

* =1 test printing, 0 no

itestxs=0

*

If(itestxs.ne.0) open(21,file='test.xs')

If(itestxs.ne.0) Write(21,1000)

1000   Format('   I       E        dSdX      dEdX_Nuc',

# '    dEdX_Nuc     dEdX_Nuc      CST'/

# '                               (tot)      [0-Ed]',

# '       [Ed-Tmax]    (barn)'/77('-'))

*

*

NK1=NK+1

*

*

* General arrays preparation

*****************************

*

*

if(istop.ne.-1) goto 3001

* Ziegler data

* ============

*

Do 2000 IK=1,NK1

*     -------------

* IK - id of the moving ion

*

NdE01tmp=NdE01(IK)

Do II=1, NdE01tmp

TMP1(II)=Ex01(IK,II)

TMP2(II)=dEdx01(IK,II)

TMP3(II)=dEdxN01(IK,II)

TMP4(II)=Rp01(IK,II)

Enddo

*

Emaxtmp=Amin1(5000.,TMP1( NdE01tmp) )

Do I=1,999999999

*

* J=(lg(E)+6)*1000+1  means that energy can changes from 10**-6 MeV

* If 10002 array dimension will be increased change also 1000 below

* and in other subroutines (i.e. MODEL, DEDX_SPAR)

RLG=Float(I-1)/1000. -6.

E=10.**RLG

* E > 5000 MeV is not considered

If(E.gt.Emaxtmp) goto 2000

! Attention. NMAX is the same for all material components, it is

* supposed that SL_LNS code which is executed before IOTA code

* was stopped at the same energy for all components

NMAX=I

If(NMAX.gt.10002) Call Error('NMAX',10002)

Call ARRAYINT(0,TMP1,TMP2, NdE01tmp,E,2, RES1)

Call ARRAYINT(0,TMP1,TMP3, NdE01tmp,E,2, RES2)

Call ARRAYINT(0,TMP1,TMP4, NdE01tmp,E,2, RES3)

dEdx(IK,NMAX)  =RES1

dEdxN1(IK,NMAX)=RES2

Rp1(IK,NMAX)   =RES3

Enddo    !  Do I1=1,999999999

2000   Continue !  Do IK=1,NK1

*

*

*

* Cross-sections obtained by Lindhard approach

* ============================================

*

3001                          continue

*

Do 3 IK=1,NK1

*     -------------

*

* IK - id of the moving ion

* MK - id of the material component

*

Do 2 MK=1,NK

*     ------------

*

If(itestxs.ne.0) Write(21,11) IK,MK

11    Format('Interaction (ion id)  ',i2,' +',i3)

*

NSL01tmp=NSL01(IK,MK)

Do II=1, NSL01tmp

TMP1(II)=Energy01(IK,MK,II)

TMP2(II)=dSdX01(IK,MK,II)

TMP3(II)=dEdX_Nuc01(IK,MK,II)

TMP4(II)=dEdX_Nuc_0Ed01(IK,MK,II)

TMP5(II)=dEdX_Nuc_Ed_Tmax01(IK,MK,II)

TMP6(II)=CST01(IK,MK,II)

Enddo

*

Emaxtmp=Amin1(5000.,TMP1( NSL01tmp) )

Do I=1,999999999

*

* J=(lg(E)+6)*1000+1  means that energy can changes from 10**-6 MeV

* If 10002 array dimension will be increased change also 1000 below

* and in other subroutines ( MODEL, DEDX_SPAR, ...)

RLG=Float(I-1)/1000. -6.

E=10.**RLG

* E > 5000 MeV is not considered

If(E.gt.Emaxtmp) goto 2

NMAX=I

If(NMAX.gt.10002) Call Error('NMAX',10002)

*

*

Call ARRAYINT(0,TMP1,TMP2, NSL01tmp,E,2, RES1)

Call ARRAYINT(0,TMP1,TMP3, NSL01tmp,E,2, RES2)

Call ARRAYINT(0,TMP1,TMP4, NSL01tmp,E,2, RES3)

Call ARRAYINT(0,TMP1,TMP5, NSL01tmp,E,2, RES4)

Call ARRAYINT(0,TMP1,TMP6, NSL01tmp,E,2, RES5)

If(itestxs.ne.0) Write(21,1)I,E,RES1,RES2,RES3,RES4,RES5

1      Format(i5,g12.5,5g12.5)

dSdX1(IK,MK,NMAX)        =RES1

dEdXNuc1(IK,MK,NMAX)     =RES2

dEdXNuc0Ed1(IK,MK,NMAX)  =RES3

dEdXNucEdTm1(IK,MK,NMAX) =RES4

* barn ---> cm2

CST1(IK,MK,NMAX)         =RES5*1.e-24

EndDo

*

*

2   Continue

3   Continue

*

If(itestxs.ne.0) close(21)

Return

End

*

************************************************************************

*

Subroutine INDICATOR(IK,T,IDE)

*       ------------------------------

Common/EKIN1/EKIN(405),HEK,MEK ! MEK can be less than 405

*

If(IK.ne.1) goto 2000

Do I=1,405

IDE=I

If(T.le.EKIN(i)) Return

Enddo

*

Print *,'SUBR. INDICATOR   T SEEMS STRANGE =',T

Call ERROR('IDE ',00000)

* to check IDE for non primary particle

2000    If(IDE.gt.0.and.IDE.le.405)   Return

* IDE <= 0 or > 405

*

Print *,'SUBR. INDICATOR:   E R R O R !   IDE =',IDE

Call ERROR('IDE ',00000)

End

*

************************************************************************

*

Subroutine Initialize_and_Zeroize(T00)

*      --------------------------------------

parameter (maxc1=22)

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

*

Common/urand1/iyg

Common/SUN/Vel1L(405),Tmean1,Vel2Tm(405),Vel3Pl(405),

#            R1,R1p,Tplay1,Vel4(405),Nel(405)

Common/Interr/Interr1

Common/MOON/XX(105),DD(105),DDF(105),TK(105),HST

COMMON/RECSTORE/TKIN(50003),ZZ(50003),NKO(50003),INDIC(50003),

#                 NREC

COMMON/MEMORY/NFRENKEL(405)

Common/CUT1/rmaxpl,itot,icut

Common/EKIN1/EKIN(405),HEK,MEK ! MEK can be less than 405

Common/SUN2/ZV5,Vel5,NN(405)

Common/SUNDI/DIVel1(405),DIFRENKEL(405),DIVel5

iyg=1

T0=T00

*

*

*                                             Ziegler Range

JE=Ifix((alog10(T0)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

*

* Ziegler Range for projectile

RZ=Rp1(1,JE)

*

RFULL=(1.+0.333333*A2/A1)*RZ     !  For effective A2

*

HST=RFULL/20.

XXX=-HST

*

Do i=1,105

XXX=XXX+HST

XX(i)=XXX

DD(i)=0.0

DDF(i)=0.0

TK(i)=0.0

enddo

*

*

MEK=200       ! should be less than 405

HEK=T0/FLOAT(MEK)

EEE=-HEK+0.5*HEK

*

Do i=1,405

EEE=EEE+HEK

EKIN(i)=EEE

enddo

*

* indent for Z where the primary energy lost = 5 %

ZV5= 0.05*T0/dEdx(1,JE)

Vel5=0.0

DIVel5=0.0

*

* Zeroize

* -------

*

Do i=1,405

Nel(i)   =0

NN(i)    =0

Vel1L(i) =0.

DIVel1(i)=0.

Vel2Tm(i)=0.

Vel3Pl(i)=0.

Vel4(i)  =0.

Enddo

*

Tmean1=0.

R1=0.

R1p=0.

Tplay1=0.0

*

Interr1=0

Icut=0

Itot=0

*

Do i=1,50003

Tkin(i)=0.0

ZZ(i)=0.0

NKO(i)=0

enddo

NREC=0

*

Do i=1,405

NFRENKEL(i)=0

DIFRENKEL(i)=0.0

Enddo

c

Return

End

*

************************************************************************

*

Subroutine INTERMEDIATE(X1,X2,Y1,Y2,WL,INTS,RESINT)

*          ---------------------------------------------------

If(INTS.eq.1) goto 1

If(INTS.eq.2) goto 2

If(INTS.eq.3) goto 3

If(INTS.eq.4) goto 4

If(INTS.eq.5) goto 5

Print *,

#     'This interpolation scheme ',INTS,' is not supported'

Print *,'                        press any key...'

pause

Stop

* Histogr

1          RESINT =  Y1

Return

* Lin-lin

2          Call REGR1(X1,X2,Y1,Y2,AAA,BBB)

RESINT =  AAA * WL + BBB

Return

* Lin(lg)-lin

3          aX1=alog10(x1)

aX2=alog10(x2)

aY1=y1

aY2=y2

aWL=alog10(wl)

Call REGR1(aX1,aX2,aY1,aY2,AAA,BBB)

RESINT =  AAA * aWL + BBB

Return

* Lin-lin(lg)

4          aX1=x1

aX2=x2

aY1=alog10(y1)

aY2=alog10(y2)

aWL=wl

Call REGR1(aX1,aX2,aY1,aY2,AAA,BBB)

RESINT = 10.**( AAA * aWL + BBB )

Return

* Lin(lg)-lin(lg)

5          aX1=alog10(x1)

aX2=alog10(x2)

aY1=alog10(y1)

aY2=alog10(y2)

aWL=alog10(wl)

Call REGR1(aX1,aX2,aY1,aY2,AAA,BBB)

RESINT = 10.**(   AAA * aWL + BBB  )

Return

End

C

C***********************************************************************

C

Subroutine REGR1(x1,x2,y1,y2,a,b)

Rr=x2-x1

A=(y2-y1)/rr

B=(y1*x2-x1*y2)/rr

Return

End

*

************************************************************************

*

Subroutine LINDINT(T00)

*      -----------------------

*

parameter (maxc=21,maxc1=22)

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

*

Common/STARS1/ dSdX1(MAXC1,MAXC,10002),

# dEdXNuc1(MAXC1,MAXC,10002),  dEdXNuc0Ed1(MAXC1,MAXC,10002),

# dEdXNucEdTm1(MAXC1,MAXC,10002), CST1(MAXC1,MAXC,10002), NMAX

Common/Edmin/Edmin ! It is not minimal displacement energy

Common/BASIC/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/SUN/Vel1L(405),Tmean1,Vel2Tm(405),Vel3Pl(405),

#            R1,R1p,Tplay1,Vel4(405),Nel(405)

*

T=T00

X=0.0

*

IK=1  ! primary particle

*

*

*                                             Ziegler Range

JE=Ifix((alog10(T)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

*

* Ziegler Range for projectile

RZ=Rp1(1,JE)

*

RFULL=(1.+0.333333*A2/A1)*RZ     !  For effective A2

*

DO 1000 ILIL=1,11111111

* Get "energy indicator number" for current T

*   IDE corresponds only to the energy of primary particle (IK=1)

Call INDICATOR(IK,T,IDE)

* general array index

JE=Ifix((alog10(T)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

*

c Define STEP for x(STEP) and dT (DT)

c (energy step is 0.01 from current energy of particles)

STOPPING=dEdx(IK,JE)

DT=0.01*T

STEP=DT/STOPPING

*

dSdx=0.0

Do MK=1,NK

dSdx=dSdx+dSdx1(IK,MK,JE)*FR(MK)

Enddo

*

Vel4(IDE)=Vel4(IDE)+dSdx*STEP

cc         Write(551,*)'T,STEP,VEL4,DT=',T,STEP,VEL4,DT

T=T-DT

*

* stop option

If(T.lt.EDmin) goto 2000

X=X+STEP

***      if(X.gt.RFULL)  goto 2000

*

*

1000     Continue

call error('str ',1000)

2000     Continue

*

Return

End

*

************************************************************************

*

Subroutine MDPARAM(IZ0,NOTE)

*       --------------------------------------------

Character NOTE*50

Common/MERCURY/CMD(5),Ecrit,MD

*

* Here are default parameters for efficiency obtained from MD

* calculations

*

* General formula for efficiency :  eff=C(1)*eMD**C(2) + C(3)*eMD

*

*  introduce

*            a) C(1)-C(3)

*            b) C(4):   eMD(keV) the lowest energy where effic=const

*            c) C(5):   eMD(keV) the highest energy where effic=const

*            d) Ecrit (keV):   eMD < Ecrit: MD;  eMD > Ecrit: BCA

*

* Note. eMD is not the incident ion energy, it is equal

*       approximately to the damage energy in the NRT formula

*

* Identification only for Z

*

If(IZ0.eq.26) goto 26

If(IZ0.eq.29) goto 29

If(IZ0.eq.74) goto 74

goto 99999

c

c Iron

c ----

c

26    NOTE='from Stoller '

CMD(1) = 0.5608

CMD(2) =-0.3029

CMD(3) = 3.227e-03

CMD(4) = 0.0

CMD(5) = 40.0

Ecrit  = 40.0

return

c

c Copper

c ------

c

29    NOTE='from Caturla et al '

CMD(1) = 0.7066

CMD(2) =-0.437

CMD(3) = 2.28E-03

CMD(4) = 0.451

CMD(5) = 20.00

Ecrit  = 20.00

return

c

c Tungsten

c --------

c

74    NOTE='from Caturla et al '

CMD(1) = 1.0184

CMD(2) =-0.667

CMD(3) = 5.06E-03

CMD(4) = 1.0

CMD(5) = 31.02

Ecrit  = 31.02

return

*

*

*

99999   Print 1,IZ0

Write(33,1)IZ0

1   Format(/' Default parameters for calculation of efficiency',

#  ' of defect production '/'           according to MD',

#  ' are ABSENT for Z=',i3,'  !!!'/20x,'Check Subr. MDPARAM !'/)

Print *,'Chiao !'

Print *,'                      press any key...'

pause

stop

end

*

************************************************************************

*

Subroutine MODEL(IK,T00,Z00,IDE)

*      --------------------------------

*

* IK  = 1 primary ion, 2,3,... =material component

* Example:

* dEdx(1,10002) is the full stopping power for the INITIAL particle +

*               composite material

* dEdx(2,10002) is the full stopping power for the FIRST component

*               of the material with the composite material  etc.

* dSdX1(1,1,10002) is the stopping power "considering defect production"

*               for the interaction of the INITIAL particle with

*               the energy transfer to FIRST material component (this

*               PKA is moving in the media with the effective Z,A (see

*               separate code SL_LNSXX, XX=ver))

* dSdX1(1,2,10002) is for the INITIAL particle and energy

*               transfer to the SECOND material component

* dSdX1(2,1,10002) is for the FIRST material component and energy

*               transfer to the same FIRST material component

* dSdX1(2,2,10002) is for the FIRST material component and energy

*               transfer to the SECOND material component

* etc.

* NK            is the number of material components (max 7)

*

*

parameter (maxc=21,maxc1=22)

Common/BASIC/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/Edmin/Edmin ! Edmin is not minimal displacement energy

Common/STARS1/ dSdX1(MAXC1,MAXC,10002),

# dEdXNuc1(MAXC1,MAXC,10002), dEdXNuc0Ed1(MAXC1,MAXC,10002),

# dEdXNucEdTm1(MAXC1,MAXC,10002), CST1(MAXC1,MAXC,10002), NMAX

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

Common/Interr/Interr1

Common/SUN/Vel1L(405),Tmean1,Vel2Tm(405),Vel3Pl(405),

#            R1,R1p,Tplay1,Vel4(405),Nel(405)

Common/MOON/XX(105),DD(105),DDF(105),TK(105),HST

Common/MEMORY/NFRENKEL(405)

Common/SUN2/ZV5,Vel5,NN(405)

Common/SATURN/TcritLAB(MAXC1)

Dimension CSS(MAXC)

*

T=T00

X=0.0  ! along the trajectory

Z=0.0  ! along the z-axis

ZD=Z00

Tcrit=TcritLAB(ik)

* general array index

JE=Ifix((alog10(T)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

*

*

*

TplH1=0.0   ! Tplay for single history

*

1      Continue      ! General loop

* Get "energy indicator number" for current T

*   IDE corresponds only to the energy of primary particle

*   and is not changed for any other generations

Call INDICATOR(IK,T,IDE)

*

If(IK.eq.1) NN(ide)=NN(ide)+1

*

*

* Estimation "elastic proton path"

* general array index

JE=Ifix((alog10(T)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

* define total cross-section

CSS(1)=FR(1)*CST1(IK,1,JE)

If(NK.eq.1) goto 2

DO MK=2,NK

CSS(MK)=CSS(MK-1)+FR(MK)*CST1(IK,MK,JE)

Enddo

2     CSelT=CSS(NK)

If(CSelT.le.0.0) goto 2000

*

dLel=1./(CSelT*RN0)  ! xsect in cm2

*

*

* STEP definition :

*

* Alternative  counting   DT0=dEdx(IK,JE)*STEP

! DT1 = only electronic component

DT1=(   dEdx(IK,JE)-dEdxN1(IK,JE)  )

! DT2 = nucl stopp power for Tk < Ed

dEdXNuc0Ed=0.0

Do MK=1,NK

dEdXNuc0Ed=dEdXNuc0Ed+dEdXNuc0Ed1(IK,MK,JE)*FR(MK)

Enddo

DT2=dEdXNuc0Ed

*

DT3=DT1+DT2

* Free path/10.

ST1=dLel/10.

ST2=1.e+20

* Change of T = 0.05 due to el.loss and nucl.loss < Ed

If(DT3.gt.0.0) ST2=0.05*T/DT3

STEP=AMIN1(ST1,ST2)

*

*

* Play if ELASTIC interaction occurs (for the transferring of the

* energy > Ed)

c

PREL=STEP/dLel

PR0=RANDOM(0)

IEL=0

COST=1.0

*

if(PR0.le.PREL)               then

*"Elastic" interaction occurs !

IEL=1

*

* Define (play) the type of the component of the composite material

PR00=RANDOM(0)

If(NK.eq.1) then

LK=1

goto 3

endif

DO MK=1,NK

LK=MK

If(PR00 .le. CSS(MK)/CSelt) goto 3

ENDDO

print *,' It is not corr play:',CSS

3     Continue

*

Tmean=dEdXNucEdTm1(IK,LK,JE)/(CSelT*RN0)

If(IK.eq.1 ) then

Nel(IDE)=Nel(IDE)+1

DISPLL=dSdx1(IK,LK,JE)/(CSelT*RN0)

Vel1L(IDE)=Vel1L(IDE)+DISPLL

Tmean1=Tmean+Tmean1

Vel2Tm(IDE)=Vel2Tm(IDE)+CF(LK+1,LK,Tmean)   ! LK

endif

* play T of PKA basing on dS/dT above Ed

TPVA=  TPKA(IK,LK,T,Tmean)

*

if(IK.eq.1)  then

* define cos of the scattering

CALL COS_DEFINE(IK,LK,T,TPVA,COST)

Tplay1=Tplay1+TPVA

Vel3Pl(IDE)=Vel3Pl(IDE)+CF(LK+1,LK,TPVA)     ! LK

endif

*

If(T.le.Tcrit) goto 7000    !   MD

*

T=T-TPVA

TplH1=TplH1+TPVA   ! Tplay for single history

*

If(TPVA.gt.Tcrit) then

CALL STORE_IN(LK,TPVA,ZD,IDE)

* number of Frenkel pairs

NFRENKEL(IDE)=NFRENKEL(IDE)+1

else  ! do not remember this particle

* MD

Frcut=0.0

If(TPVA.gt.0.0)   FRcut=CF2(LK+1,LK,TPVA) +0.5

NFRENKEL(IDE)=NFRENKEL(IDE)+Ifix(FRcut)

endif

*

If(T.le.Tcrit) goto 7000    !   MD

*

*

If(ZD.le.ZV5) Vel5=Vel5+1.

*

Do IR=1,105

If(ZD.le.XX(IR)) then

DDF(IR)=DDF(IR)+ 1.  ! Direct summing

goto 499

endif

enddo

*

*

499     If(IK.ne.1) goto 510

Do IR=1,105

If(Z.le.XX(IR)) then

DD(IR)=DD(IR)+ DISPLL  ! Lindhard component is summing

goto 500

endif

enddo

endif !  if(PR0.le.PREL)

500    continue

*End ---elastic

*

Do IR=1,105

If(Z.le.XX(IR)) then

TK(IR)=TK(IR)+ T*STEP

goto 510

endif

enddo

*

510     continue

*

If(T.le.Tcrit) goto 7000    !   MD

*

T=T-DT3*STEP

*

If(T.le.Tcrit) goto 7000    !   MD

*

* usual end of history

If(T.lt.EDmin) goto 2000

X=X+STEP

DZ=STEP

If(IEL.ne.0) DZ=STEP*COST

Z=Z+DZ

ZD=ZD+DZ

Goto 1

*

2000     If(IK.eq.1) then

R1=R1+X

R1p=R1p+Z

endif

Return

*

* Interrupt history taking MD results for the rest energy

7000    continue

*no alloy

LK=1

Frcut=0.0

If(T.gt.0.0)   FRcut=CF2(IK,LK,T) +0.5

NFRENKEL(IDE)= NFRENKEL(IDE)+Ifix(FRcut)

goto 2000

End

*

************************************************************************

*

Subroutine NRT_COEFF(Z1,Z2,A1,A2,ANRT,BNRT,GNRT)

*         -----------------------------------------------

PI=3.1415926

A0=0.52918E-08    ! Bohr radius, cm

EL=4.8E-10        ! electron charge

C Classical NRT, Nucl.Eng.Des., 1975 :

C         RK0=0.1337*(Z1**(1./6.))*SQRT(Z1/A1)

C Robinson

RK0=0.0793*(Z1**0.6666666)*SQRT(Z2)*((A1+A2)**1.5)/         (

#    (A1**1.5)*SQRT(A2)*((Z1**0.666666666+Z2**0.66666666)**0.75) )

C

AA=A0*(  (9.*Pi*Pi/128.)**0.333333333  )/

#          SQRT(Z1**0.6666666+Z2**0.6666666)

E0=Z1*Z2*(EL**2)*(A1+A2)/(AA*A2)

E0=E0/1.6E-06    !  erg ---> MeV

E0=1000.*E0      !      ---> keV

EE=1./E0

C

ANRT=RK0*EE

BNRT=0.40244*RK0*(EE**0.75)

GNRT=3.4008*RK0*(EE**0.1666666666)

RETURN

END

*

************************************************************************

*

Subroutine PRECALCULATION

*       -------------------------

parameter (maxc=21,maxc1=22,maxe=50001)

Character INPNAM_XS*12, INPNAM_DEDX*12, OUTNAM*12

Character CTASK_NAME*80, C50*50

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/PKA/ALPHA2(MAXC1,MAXC),E0(MAXC1,MAXC)

COMMON/NRT1/ANRT(MAXC1),BNRT(MAXC1),GNRT(MAXC1)

Common/Edmin/Edmin

Common/PARAM01/IDSDT

Common/FILENAMES/INPNAM_XS(MAXC1,MAXC),INPNAM_DEDX(MAXC1),

#  OUTNAM

Common/MERCURY/CMD(5),Ecrit,MD

Common/DATATAB1/WTAB(101),ROTAB(101),RITAB(101),IATAB(101)

Common/ISTOP/ISTOP

Common/VENUS/RI(MAXC)

Common/EARTH/IDEFMD,CTASK_NAME,C50

Common/AUX1/rlambdaft12,rmft12,qft12

Common/SATURN/TcritLAB(MAXC1)

Common/EIN/EIN(MAXE),NH(MAXE),KEIN

Dimension TmpEn(MAXE)

*

AVO=6.022E+23     ! Avogadro

IZ1=ifix(Z1+0.001)

*

* Get masses, density and <I>

Call DATATAB

*

* Get projectile A, if absent

If(A1.le.0.0) then

A1=WTAB(IZ1)

endif

If(A1.le.0.0) Call Error(' A1 ',000)

IA1=ifix(A1+0.001)

*

* Get A for material components, if absent

do i=1,nk

If(AT(i).le.0.0) then

IZT=ifix(ZT(i)+0.001)

AT(i)=WTAB(IZT)

endif

If(AT(i).le.0.0) Call Error(' AT ',i)

enddo

*

* Density

if(RO.le.0.0) then

IZT=ifix(ZT(NK)+0.001)  ! check for NK=1 in sub read_inp

RO=ROTAB(IZT)

endif

If(RO.le.0.0) Call Error(' RO ',NK)

*

* Mean ionization potential for SPAR calculation, if chosen

if(istop.eq.1) then

do i=1,nk

IZT=ifix(ZT(i)+0.001)

RI(i)=RITAB(IZT)*1.E-6  ! eV --> MeV (SPAR)

If(RI(i).le.0.0) Call Error(' RI ',i)

enddo

endif

*

*

* Normalize fractions on unity

sum=0.0

do i=1,nk

sum=sum+FR(i)

enddo

Do i=1,nk

FR(I)=FR(I)/SUM

Enddo

*

* Get effective Z and A

Z2=0.0

A2=0.0

Do i=1,nk

Z2=Z2+FR(I)*ZT(I)

A2=A2+FR(I)*AT(I)

Enddo

AW=A2

*

* Atoms (molecules)

RN0=AVO*RO/AW     ! at/cm3

DA=(1./RN0)**0.33333333  ! ave dist betw at (not used)

*

* Min ED for the composite material

* ! Edmin is not minimal displacement energy

Edmin=1.E+10

do i=1,nk

If(ED(i).lt.Edmin) Edmin=ED(i)

enddo

*

* omit input energies < Edmin

ke=0

Do i=1,KEIN

If(EIN(i).ge.Edmin) then

ke=ke+1

TmpEn(ke)=EIN(i)

endif

Enddo

Do i=1,ke

EIN(i)=TmpEn(i)

Enddo

KEIN=KE

*

NK1=NK+1

*

* NRT coefficients for effective material Z,A

Do IK=1,NK1

If(IK.eq.1) then

Z111=Z1

A111=A1

endif

If(IK.ne.1) then

Z111=ZT(IK-1)

A111=AT(IK-1)

endif

Call NRT_COEFF(Z111,Z2,A111,A2,ATRN,BTRN,GTRN)

ANRT(IK)=ATRN

BNRT(IK)=BTRN

GNRT(IK)=GTRN

Enddo

*

* For joint BCA-MD calculations define "critical" energy for each ion

If(MD.eq.1) Call CRITICAL_ENERGY

*

*

* Data for Lindhard F(x) function

* -------------------------------

PI=3.1415926

EL=4.8E-10        ! electron charge

A0=0.52918E-08    ! cm

*

* IK =1, 2...  1 = projectile, other material component

* MK =1, 2...    = material component

*

*

DO IK=1,NK1

If(IK.eq.1) then

Z111=Z1

A111=A1

endif

If(IK.ne.1) then

Z111=ZT(IK-1)

A111=AT(IK-1)

endif

DO MK=1,NK

Z222=ZT(MK)

A222=AT(MK)

c

* LNS:

AA=A0*(  (9.*Pi*Pi/128.)**0.333333333  )

#         /SQRT(Z111**0.6666666+Z222**0.6666666)

* Firsov

If(IDSDT.ne.0)

#    AA=A0*(  (9.*Pi*Pi/128.)**0.333333333  )

#         /(   ( SQRT(Z111)+SQRT(Z222) )**0.666666666   )

*

*

E0(IK,MK)=Z111*Z222*(EL**2)*(A111+A222)/(AA*A222)

E0(IK,MK)=E0(IK,MK)/1.6E-06    !  erg ---> MeV

ALPHA2(IK,MK)=4.*A111*A222/((A111+A222)**2)

PIA2=Pi*(AA**2)

ENDDO

ENDDO

*

*********************************************************************

* open main output file and write basic information

*

open(33,file=OUTNAM)

Write(33,*)' I O T A   C O D E'

Write(33,'(a80)')CTASK_NAME

Write(33,*)' '

*

Write(33,1000)IZ1,A1

1000   Format(1x,'Primary particle (Z,A)',i3,f8.2)

Write(33,1001)

1001   Format(' Material      at %    Ed (eV)')

Do IK=1,NK

IZ=Ifix(ZT(IK)+0.0001)

FR0=FR(IK)*100.

Write(33,1002)IZ,AT(ik),FR0,ED(IK)*1.e+06

1002   Format(i3,f8.2,f9.2,f9.1)

Enddo

If(IDSDT.eq.0) Write(33,1003)rlambdaft12,rmft12,qft12

1003   Format(' F(t1/2) function: LNS   L=',f6.3,

#  '  m=',f6.3,'  q=',f6.3)

If(IDSDT.ne.0) Write(33,1004)

1004   Format(' F(t1/2) function: Burenkov et al')

*

If(MD.eq.1)      then

Write(33,701)

701          Format(/t27,'Joint BCA-MD simulation')

If(IDEFMD.eq.1) Write(33,7011)C50

7011         Format(' default parameters   ',a50)

*

if(ifix(ZT(1)+0.001).eq.IZ1.and.ifix(AT(1)+0.001).eq.IA1) then

Write(33,702)TcritLAB(1)*1000.

702     Format(' MD below Tcrit =',f6.2,' keV')

else

if(TcritLAB(1)*1000..lt.1.e+3) then

Write(33,7021)TcritLAB(1)*1000.,TcritLAB(2)*1000.

else

Write(33,7022)TcritLAB(1)*1000.,TcritLAB(2)*1000.

endif

7021    Format(' MD below Tcrit =',f6.2,' keV (projectile); =',f6.2,

#  ' keV (ion of target)')

7022    Format(' MD below Tcrit =',g12.5,' keV (projectile); =',f6.2,

#  ' keV (ion of target)')

If(TcritLAB(1).ge.100.) Write(33,7023)TcritLAB(1)

7023    Format(70('*')/'*         Energy Tcrit =',g12.5,' keV',

#  ' is not realistic',T70,'*'/70('*'))

endif

*

Write(33,703)(CMD(i),i=1,3)

703          Format(' efficiency(MD)=',f7.4,'*( eMD**(',f8.4,') ) +',

#       1pe10.3,'*eMD')

Write(33,704)CMD(4),CMD(5)

704          Format(' constant effciency below',f7.3,' keV and above',

#       f7.2,' keV'/)

endif

If(istop.eq.-1) Write(33,*)' Stopping power: Ziegler, SRIM'

If(istop.eq. 1) Write(33,*)' Stopping power: Armstrong, SPAR'

*

*

Write(33,1005)RO,RN0

1005     Format(' Density =',f8.4,' g/cm3    n0=',1pe12.5,' 1/cm3')

Write(33,1006)Z2,A2

1006     Format(' Z(eff) =',f6.2,'    A(eff)=',f7.2)

Write(33,1007)

1007     Format(' Note. Nuclear nonelastic interactions ',

#    'are not considered here')

*

Return

End

*

************************************************************************

*

Subroutine PRINT(T0,Nhist)

*      --------------------------

parameter (maxc=21,maxc1=22)

Character INPNAM_XS*12, INPNAM_DEDX*12, OUTNAM*12

Common/FILENAMES/INPNAM_XS(MAXC1,MAXC),INPNAM_DEDX(MAXC1),

# OUTNAM

*

Common/BASIC/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/SKY1/dEdx(MAXC1,10002),dEdxN1(MAXC1,10002),

# Rp1(MAXC1,10002)

Common/SUN/Vel1L(405),Tmean1,Vel2Tm(405),Vel3Pl(405),

#            R1,R1p,Tplay1,Vel4(405),Nel(405)

Common/STARS1/ dSdX1(MAXC1,MAXC,10002),

# dEdXNuc1(MAXC1,MAXC,10002), dEdXNuc0Ed1(MAXC1,MAXC,10002),

# dEdXNucEdTm1(MAXC1,MAXC,10002), CST1(MAXC1,MAXC,10002), NMAX

Common/Interr/Interr1

Common/MOON/XX(105),DD(105),DDF(105),TK(105),HST

COMMON/MAIIPR/IPR1

COMMON/MEMORY/NFRENKEL(405)

Common/CUT1/rmaxpl,itot,icut

Common/OPTIONS1/igraph,maxplay

Common/EKIN1/EKIN(405),HEK,MEK ! MEK can be less than 405

Common/SUN2/ZV5,Vel5,NN(405)

Common/SUNDI/DIVel1(405),DIFRENKEL(405),DIVel5

Common/MERCURY/CMD(5),Ecrit,MD

Common/ISTOP/ISTOP

Dimension EKIN2(405)

*

* Output file is opened in Subr. Read_input

*

JE=Ifix((alog10(T0)+6.)*1000.)+1

If(JE.le.0.or.JE.gt.10002) Call Error(' JE ',11111)

* SRIM or SPAR range for projectile

RZ=Rp1(1,JE)

if(istop.eq.-1) RFULL=(1.+0.333333*A2/A1)*RZ     !  for effective A2

if(istop.eq. 1) RFULL=RZ

*

RNel=0.0

Do i=1,405

RNel=RNel+Float(Nel(i))/Float(Nhist)

Enddo

*

Do i=1,405

Vel1L(i) =Vel1L(i) /Float(Nhist)

Vel2Tm(i)=Vel2Tm(i)/Float(Nhist)

Vel3Pl(i)=Vel3Pl(i)/Float(Nhist)

Enddo

*

Vel5=Vel5/Float(Nhist)

* Sum

Vel1LS =0.0

Vel2TmS=0.0

Vel3PlS=0.0

Vel4S  =0.0

Do i=1,405

Vel1LS =Vel1LS +Vel1L(i)

Vel2TmS=Vel2TmS+Vel2Tm(i)

Vel3PlS=Vel3PlS+Vel3Pl(i)

Vel4S  =Vel4S  +Vel4(i)

Enddo

*

* delta = Root[ M(ksi**2) -M**2 ] /M

DIS1= DIVel1(405)/Float(Nhist) -Vel1LS**2

*

DIS1= DIS1/Float(Nhist)

DEL11=SQRT(  DIS1  )

ERR1=100.

If(Vel1LS.ne.0.0) ERR1=100.*DEL11/Vel1LS

*

Tmean1=Tmean1/Float(Nhist)

Tplay1=Tplay1/Float(Nhist)

R1=R1/Float(Nhist)

R1p=R1p/Float(Nhist)

*

If(MD.eq.1) then

Write(33,701)

701     Format(35(1H*),'BCA+MD',37(1H*))

else

Write(33,1)

1       Format(78(1H*))

endif

Write(33,2)T0, Nhist

2       Format(1x,'Primary energy ',1pg12.5,' MeV'/1x,

#  ' Number of events ',i7) !1x,60('-'))

*

CFNRT=0.0

Do MK=1,NK

CFNRT=CFNRT+CF(1,MK,T0)*FR(MK)

Enddo

*

Write(33,*)'                             ',

#  '                           error %      /NRT '

Write(33,30)CFNRT,CFNRT/CFNRT

30      Format(1x,

#  'Simple NRT-formula prediction               =',g12.5

#  ,t69,f6.3,'  (1)')

*

If(MD.eq.0) Write(33,3)Vel1LS,ERR1, Vel1LS/CFNRT

3       Format(1x,

#'Number of vacancies (Lindhard, free path )  =',f8.2,

# '  (',f6.2,')'

#  ,t69,f6.3,'  (2)')

*

Write(33,31)Vel4S,Vel4S/CFNRT

31      Format(1x,

#'Number of vacancies (Lindhard, integration) =',f8.2

#  ,t69,f6.3,'  (3)')

*

FRENK1=0.0

Do i=1,405

FRENK1=FRENK1+float(NFRENKEL(i))/float(Nhist)

Enddo

*

* delta

DISF=  DIFRENKEL(405)/Float(Nhist) - FRENK1**2

DISF=  DISF/Float(Nhist)

DELF=SQRT(  DISF )

ERRF=100.

If(FRENK1.ne.0.0) ERRF=100.*DELF/FRENK1

Write(33,32)

32      Format(76('-'))

Write(33,9)FRENK1,ERRF,FRENK1/CFNRT

9       Format(1x,

#  'Number of Frenkel pairs (direct counting)   =',f8.2

#  ,'  (',f6.2,')'

#  ,t69,f6.3,'  (4)')

*

* unit=555

If(MD.ne.0) Write(55,555)

#  T0, FRENK1/CFNRT, (ERRF/100.)*FRENK1/CFNRT, FRENK1,

#                    (ERRF/100.)*FRENK1,       CFNRT

555     Format(1pg12.5,  0pf8.3, f8.3, f14.2, f9.2, f14.2)

*

Write(33,32)

*

* "Modified" counting = direct count *0.8 ( "as in NRT" )

If(MD.eq.0) Write(33,10)0.8*FRENK1, ERRF, 0.8*FRENK1/CFNRT

10      Format(1x,

#  'Modified counting                           =',f8.2

#  ,'  (',f6.2,')'

#  ,t69,f6.3/)

*

CFTRN=0.0

*

* NRT formula for pure material weighted with given fractions

If(NK.eq.1) goto 1000

*

CFTRN=0.0

Do MK=1,NK

Z222=ZT(MK)

A222=AT(MK)

ED2=ED(MK)

Call NRT_COEFF(Z1,Z222,A1,A222,ATRN,BTRN,GTRN)

CFTRN=CFTRN+FR(MK)*CF1(ED2,ATRN,BTRN,GTRN,T0)

Enddo

Write(33,11)CFTRN

11      Format(1x,

#  'NRT-formula for pure material components weighted =',f8.2

#  ,T69,'      ','  (5)')

1000   continue

*

If(MD.eq.0) Write(33,4)Vel2TmS,Vel2TmS/CFNRT

4       Format(1x,

#  'Number of vacancies from Tmean calc               =',f8.2

#  ,t69,f6.3)

*

If(MD.eq.0) Write(33,5)Vel3PLS,Vel3PLS/CFNRT

5       Format(1x,

#  'Number of vacancies from Tplay calc               =',f8.2

#  ,t69,f6.3)

*

* Get displacement cross-sections

* Lindhard, code SL_LNS :

CSDL=0.0

Do MK=1,NK

CSDL=CSDL+FR(MK)*dSdx1(1,MK,JE)/RN0

Enddo

CSDL=CSDL*1.e+24

CSDDIR=1.e+24*Vel5/(RN0*ZV5)

*

* delta

DISCS= DIVEL5/Float(Nhist) - Vel5**2

DISCS= DISCS/Float(Nhist)

DELCS=SQRT(  DISCS )

ERRCS=100.

If(CSDDIR.ne.0.0.and.Vel5.ne.0.0) ERRCS=100.*DELCS/Vel5

*

If(NK.eq.1) Write(33,51)CSDL

51    Format(/

#  ' Displacement cross-section (Lindhard, integr)   (b) =',

#  1pg12.5,

#  T69,'      ','  (6)')

If(NK.ne.1) then

Write(33,52)CSDL

52    Format(/

#  ' Displacement cross-section (Lindhard, weighted) (b) =',

#  1pg12.5,

#  T69,'      ','  (6)')

Write(33,53) (dSdx1(1,MK,JE)/(1.e-24*RN0),MK=1,NK)

53    Format(' Components: ',1pg12.5,4g12.5)

endif

If(MD.eq.0) Write(33,54)CSDDIR,ERRCS

54    Format(

#  ' Displacement cross-section (direct counting)    (b) =',

#  1pg12.5,  ' (',  0pf6.2,   ') (7)' )

*

If(MD.eq.0) Write(33,55)0.8*CSDDIR,ERRCS

55    Format(

#  '                            (modified count )    (b) =',

#  1pg12.5,  ' (',  0pf6.2,   ')    '/)

*

* unit=44

If(MD.eq.0) then

Write(44,111)T0,CFNRT,Vel1LS,Vel4S,FRENK1,CFTRN,CSDL,CSDDIR

111    Format(1pg12.5,   0pf10.2,  4f10.2,1pg12.5,g12.5)

else

Write(44,111)T0,CFNRT, 0.0  ,Vel4S,FRENK1,CFTRN,CSDL, 0.0

endif

*

*

*

If(MD.eq.0) Write(33,6)Tmean1,Tplay1

6       Format(1x,'Tmean =',1pe12.5,' MeV     Tplay=',g12.5)

*

If(MD.eq.0) then

if(istop.eq.-1) Write(33,7)RZ,RFULL,R1

7    Format(

#' Path (Ziegler,evl)=',1pe10.3,' cm,  total=',e10.3,' cm'/

#' Path (total,calc )                      =',e10.3,' cm')

if(istop.eq.1) Write(33,8)RFULL,R1

8    Format(' Total path (SPAR)=',1pe10.3,' cm',

#'   path (total,calc)=',e10.3,' cm')

else

if(istop.eq.-1) Write(33,17)RZ

17    Format(' Path (Ziegler,evl)=',1pe10.3,' cm')

if(istop.eq.1) Write(33,18)RFULL

18    Format(' Total path (SPAR)=',1pe10.3,' cm')

endif

*

*

Write(33,81)Interr1

81      Format(1x,' Histories interrupted (subr model)=', i12)

tpc=0.0

if(itot.ne.0) tpc=100.*float(icut)/float(itot)

*

If(maxplay.gt.1) then

Write(33,82)tpc,maxplay

82    Format(1x,' T play was cut (TPKA rout) =',f16.3,

#  '  %    Maxplay=',i9)

endif

If(maxplay.lt.0) then

Write(33,821)tpc

821    Format(1x,' T play was cut (TPKA rout) =',f16.3,

#  '  %    no limitation on Maxplay')

endif

If(tpc.gt.5.0.and.maxplay.gt.1) write(33,83)tpc

83    Format(26('*'),' W A R N I N G ',30('*')/'*',

#  f15.3,' is too much,  increase Maxplay in input',

#  ' file         *'/26('*'),' W A R N I N G ',30('*'))

*

* GRAPHs preparation

*

Sum=0.0

Sumf=0.0

Sumt=0.0

do i=1,105

*

DD(i)=DD(i)/float(Nhist)

DDF(i)=DDF(i)/float(Nhist)

c recalculation to the fluens 1/cm2

DD(i)=DD(i)/(HST*RN0)

DDF(i)=DDF(i)/(HST*RN0)

*

TK(i)=TK(i)/(HST*float(Nhist))

sum=sum+DD(i)

sumf=sumf+DDF(i)

sumt=sumt+TK(i)

enddo

*

* check the maximal i-number, where DD .ne. 0

do i=1,105

j=105-i+1

if(DD(j).ne.0.0) goto 100

enddo

j=105

100    if(j.le.105-2) j=j+2

jddmax=j

*

c

c For BCA-MD no damage profile

If(MD.eq.1.and.igraph.ne.0) then

Write(33,101)

101    Format(/' (no damage profiles for joint BCA-MD calculations)')

igraph=0

endif

If(igraph.eq.0) goto 2000

*

IPR1=33

DD(1)=0.0

Write(33,12)

12    Format(/

#  ' Profile of damage from Lindhard dS/dx (free path )')

*

Write(33,13)

13    Format(

# ' X=(cm)  Y=(dpa), result is normalized on primary ',

# 'ion fluence 1/cm**2')

*

Call GRAPH(XX,DD,jddmax,0)

*

Write(33,14)Sum,Sum*(HST*RN0)

14    Format(

# ' Sum(Y)=',1pe13.5,' dpa','    (unnormalized)= ',g12.5/)

*

Write(33,15)

15    Format(/

#  ' Profile of damage from direct counting            ')

Write(33,13)

*

Call GRAPH(XX,DDF,jddmax,0)

Write(33,14)Sumf,Sumf*(HST*RN0)

*

*

2000    continue

*

* Get cascade function for different primary energies

*

* energy corresponds to the middle of interval

Do i=2,405

EKIN2(i)=0.5*( EKIN(i)+ EKIN(i-1) )

Enddo

EKIN2(1)=EKIN(1)/2. ! (see subr.Initialize_)

*

MEK1=MEK+1

If(MD.eq.0) then

Write(33,16)

16    Format(' Number of vacancies at different',

#' primary energies from current calculation'/

#'                                   ',

#'    [explanation (2),(3),(4) see above]'/75('-')/

#'    Energy     dS/dx(free path)  ',

#' dS/dx(integr)     Direct counting'/

#'     (MeV)          (2)       ',

#  '        (3)               (4)')

Vel1LS2 =0.0

Vel4S2  =0.0

FRENK2  =0.0

Do i=1,MEK1

Vel1LS2 =Vel1LS2 +Vel1L(i)

Vel4S2  =Vel4S2  +Vel4(i)

FRENK2  =FRENK2  +float(NFRENKEL(i))/float(Nhist)

Write(33,19) EKIN2(i),Vel1LS2, Vel4S2,FRENK2

19    Format(1pg12.4,4x,     0pg12.5, 2(6x,g12.5))

Enddo

endif ! If(MD.eq.0)

*

Write(33,*)' '

Write(33,*)' '

Return

End

*

************************************************************************

*

Function RANDOM(NO ARGUMENT)

*      --------------------------------

*

* random number generator

*

**** GNU Fortran pseudo random generator:

****

****

RANDOM=RAND(0)

**

**

** WATCOM Fortram pseudo random number generator

** generator is initialized in Subroutine Initialize_and_Zeroize

** See   Common/urand1/iyg;    iyg=1

**

**      Common/urand1/iyg

**      RANDOM=URAND(iyg)

**

Return

End

*

************************************************************************

*

Subroutine READ_DATA

*       --------------------

parameter (maxc=21,maxc1=22)

Character INPNAM_XS*12, INPNAM_DEDX*12, OUTNAM*12

Common/FILENAMES/INPNAM_XS(MAXC1,MAXC),INPNAM_DEDX(MAXC1),

#  OUTNAM

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

*

Common/dEdxMain01/Ex01(MAXC1,1501), dEdx01(MAXC1,1501),

#  Rp01(MAXC1,1501), dEdxN01(MAXC1,1501),NdE01(MAXC1)

*

Common/Lindhard01/Energy01(MAXC1,MAXC,1701),

#  dSdX01(MAXC1,MAXC,1701),

#  dEdX_Nuc01(MAXC1,MAXC,1701), dEdX_Nuc_0Ed01(MAXC1,MAXC,1701),

#  dEdX_Nuc_Ed_Tmax01(MAXC1,MAXC,1701),

#  CST01(MAXC1,MAXC,1701), NSL01(MAXC1,MAXC)

*

Common/istop/istop

*

Dimension Ex01tmp(1501), dEdx01tmp(1501),Rp01tmp(1501),

#  dEdxN01tmp(1501) ! NdE01tmp

c

NK1=NK+1

if(istop.ne.-1) goto 2

*

* istop=-1

*

* Reading dE/dx from SRIM/Ziegler OUTPUT

* ---------------------------------------

Do IK=1,NK1

*

IR=21

open(IR,file=INPNAM_DEDX(IK))

read(ir,*,end=90003)

backspace ir

*

Call dEdX_ZIEGLER_READ(IK, IR, RO, Ex01tmp, dEdx01tmp, Rp01tmp,

#                       NdE01tmp,   dEdxN01tmp)

* Now units for range and stopping power are Rp(cm), Ex(MeV), dEdx(MeV/cm)

NdE01(IK)=NdE01tmp

* fill arrays

do J=1,NdE01tmp

Ex01(IK,J)   =Ex01tmp(J)

dEdx01(IK,J) =dEdx01tmp(J)

Rp01(IK,J)   =Rp01tmp(J)

dEdxN01(IK,J)=dEdxN01tmp(J)

enddo

*

close(IR)

Enddo ! Do IK=1,NK1

2    continue

*

* Reading data prepared by SL_LNSxx code

* --------------------------------------

Do IK=1,NK1

*

if(IK.eq.1) then

IZP=ifix(Z1+0.001)

IAP=ifix(A1+0.001)

else

IZP=ifix(ZT(IK-1)+0.001)

IAP=ifix(AT(IK-1)+0.001)

endif

*

* IK - id of the moving ion

* MK - id of the material component

*

Do MK=1,NK

IZC=ifix(ZT(MK)+0.001)

IAC=ifix(AT(MK)+0.001)

IR=21

open(IR,file=INPNAM_XS(IK,MK))

Read(IR,*,end=90001)Z10,A10,Z20,A20

IZ10=ifix(Z10+0.001)

IA10=ifix(A10+0.001)

IZ20=ifix(Z20+0.001)

IA20=ifix(A20+0.001)

If(IZ10.ne.IZP .or. IA10.ne.IAP .or.

#     IZ20.ne.IZC .or. IA20.ne.IAC) goto 90002

do iii=1,MAXC

read(IR,*)

enddo

*

NSL01tmp=1

Energy01(IK,MK,NSL01tmp)=0.0

dSdX01(IK,MK,NSL01tmp)  =0.0

dEdX_Nuc01(IK,MK,NSL01tmp)=0.0

dEdX_Nuc_0Ed01(IK,MK,NSL01tmp)=0.0

dEdX_Nuc_Ed_Tmax01(IK,MK,NSL01tmp)=0.0

CST01(IK,MK,NSL01tmp)=0.0

*

Do iii=1,111111111

* 1    2     3               4               5                  6

* T0,  DSDX, DEDX_nuc_total, DEDX_nuc_0_Ed,  DEDX_nuc_Ed_Tmax,  CS_Ed_Tmax

Read(IR,*,end=4000) XXX1, XXX2, XXX3, XXX4, XXX5, XXX6

NSL01tmp=NSL01tmp+1

If(NSL01tmp.gt.1701) Call Error('Dime',1701)

*

Energy01(IK,MK,NSL01tmp)=XXX1

dSdX01(IK,MK,NSL01tmp)  =XXX2

dEdX_Nuc01(IK,MK,NSL01tmp)=XXX3

dEdX_Nuc_0Ed01(IK,MK,NSL01tmp)=XXX4

dEdX_Nuc_Ed_Tmax01(IK,MK,NSL01tmp)=XXX5

CST01(IK,MK,NSL01tmp)=XXX6

*

Enddo ! Do iii=1,111111111

Call Error('????',IR)

*

4000    continue

*

NSL01(IK,MK)=NSL01tmp

*

close(IR)

*

Enddo !   Do MK=1,NK

Enddo !   Do IK=1,NK1

*

check if data are prepared by SL_LNS code up to the same energy

Do IK=1,NK1

Do MK=1,NK

If(NSL01(IK,MK).ne.NSL01(1,1)) goto 90004

enddo

enddo

*

Return

*

90001   Print *,' File with the cross-sections from SL_LNS ',

#  ' code is ABSENT'

print *,' for ',IZP,IAP,' +',IZC,IAC,' interactions !'

print *,'                       press any key...'

pause

stop

90002   Print 90012,INPNAM_XS(IK,MK),IZP,IAP,IZC,IAC,Z10,A10,Z20,A20

90012   Format(1x,' Input file with the cross-sections ',a12/1x,

#  ' for ',i3,i5,' +',i3,i5,' interactions '/1x,

#  '     contains the information '/1x,

#  ' for ',2f6.1,' +',2f6.1,' interactions - E R R O R')

print *,'                       press any key...'

pause

stop

90003   Print 90013,INPNAM_DEDX(IK)

90013   Format(//30x,'E  R  R  O  R'

#  //' There are NO input file from SRIM (Ziegler)',

#  ' code '//1x,'                          NAME =  ',a12//)

print *,'                       press any key...'

pause

stop

90004   Print 90014

90014   Format(//30x,'E  R  R  O  R'

#  //' Data prepared by SL_LNS code for different components ',

#  ' do not coincide !'//1x,'  Delete corresponding files and ',

#  ' run SL_LNS again.'//)

print *,'                                     press any key...'

pause

stop

End

*

************************************************************************

*

Subroutine READ_INPUT

*       ---------------------

* + write general information in output file

c

parameter (maxc=21,maxc1=22,maxe=50001)

Character CTASK_NAME*80

Character INPNAM_XS*12, INPNAM_DEDX*12, OUTNAM*12, OUTNAM2*12

Character OUTNAM3*12

Character NF(MAXC)*12,  C50*50, C80*80

Common/FILENAMES/INPNAM_XS(MAXC1,MAXC),INPNAM_DEDX(MAXC1),

#  OUTNAM

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/EIN/EIN(MAXE),NH(MAXE),KEIN

Common/OPTIONS1/igraph,maxplay

Common/CUT1/rmaxpl,itot,icut

Common/PARAM01/IDSDT

Common/MERCURY/CMD(5),Ecrit,MD

Common/ISTOP/ISTOP

Common/EARTH/IDEFMD,CTASK_NAME,C50

Common/SATURN/TcritLAB(MAXC1)

Print *,' Wait a bit...'

c

open(1,file='input')

c

c

c find line with problem name

call star(1)

read(1,'(a80)')CTASK_NAME

c

c find line with graph printing option and maxplay

call star(1)

c

c dS/dT

c IDSDT=0, 10(=0), 11-15 LNS expression;

c      =1                Burenkov et al

c Tlim maximal energy for SL_LNS code

read(1,*)igraph,maxplay,Tlim,IDSDT

if(maxplay.eq.0) maxplay=250000  ! default option

If(Tlim.gt.5200.)then

Print *,'           E R R O R  in INPUT file       '

Print *,'Maximal energy should be below 5.2 GeV  !'

Print *,'                       press any key...'

pause

stop

endif

c

If(Tlim.le.0.0) Tlim=5000.

c

c Define f(t^1/2) function parametrs

Call FT12CHOICE(IDSDT)

c

c

If(IDSDT.ne.0.and.IDSDT.ne.1) then

Print *,'           E R R O R  in INPUT file       '

Print *,'  IDSDT parameter is incorrect'

Print *,'                          press any key...'

pause

stop

endif

c

rmaxpl=float(maxplay)

if(maxplay.lt.0) rmaxpl=1.e+9

c

c try to find line with BCA-MD instruction

call star(1)

read(1,'(a80)')C80

c "BCA-MD" is identified by "B" or "M" letter

if(findp(C80,'B').ne.0. .or. findp(C80,'M').ne.0. .or.

#     findp(C80,'b').ne.0. .or. findp(C80,'m').ne.0. ) then

c

MD=1

c find information needed for BCA-MD joint calculations

call star(1)

c

c general formula for efficiency :  eff=C(1)*eMD**C(2) + C(3)*eMD

c if T(keV) < CMD(4) eff=const   (see function CF2)

c          and

c if T(keV) > CMD(5) eff=const

read(1,*,err=9919) (CMD(i),i=1,5),Ecrit

IDEFMD=0

if(CMD(1).eq.0.0.and.CMD(2).eq.0.0.and.CMD(3).eq.0.0.and.

#  CMD(4).eq.0.0.and.CMD(5).eq.0.0.and.Ecrit.eq.0.0) IDEFMD=1

c

if(IDEFMD.eq.0) then   ! no default MD parameters

if(CMD(4).lt.1. .or. Ecrit.lt.1.0)      Call Error(' keV',1)

if(CMD(4).gt.1000. .or. Ecrit.gt.1000.) Call Error(' keV',1000)

if(CMD(1).eq.0.0.and.CMD(2).eq.0.0.and.CMD(3).eq.0.0.and.

#  CMD(4).ne.0.0.or.CMD(5).ne.0.0.or.Ecrit.ne.0.0)

#                                           Call Error('MDde',0)

c

endif

else

MD=0

Do itcritlab=1,maxc1

TcritLAB(itcritlab)=-1.E+6

Enddo

backspace 1

endif

c

c find line with Z1, A1 of projectile

call star(1)

read(1,*,err=9929)Z1,A1

c

c find line with RO (density of the material), number of material components

c and option to calculate dE/dx

call star(1)

read(1,*)RO, NK

backspace 1

istop=0

read(1,'(a80)')C80

c

c Ziegler = SRIM (identified by "Z", "I")

if(findp(C80,'Z').ne.0.0.or.findp(C80,'z').ne.0.0

#  .or.findp(C80,'I').ne.0.0.or.findp(C80,'i').ne.0.0) istop=-1

c

c Armstrong=SPAR (identified by "A")

if(findp(C80,'A').ne.0.0.or.findp(C80,'a').ne.0.0)  istop=+1

c

If(istop.eq.0) then

Print *,' '

Print *,' Model to obtain dE/dx is not identified !!'

print *,'                                press any key...'

pause

stop

endif

c

If(NK.gt.MAXC) then

Print *,'Number of components > ',MAXC,' !'

print *,'                       press any key...'

pause

stop

endif

c

If(RO.le.0. .and. NK.ne.1) then

Print *,' '

Print *,

#  '      NO  default density for compounds !!!'

print *,'                           press any key...'

pause

stop

endif

c

If(MD.ne.0 .and. NK.ne.1) then

Print *,

#  'Joint BCA-MD calculations are not tested for compounds yet !'

Print *,

#  '  (!!!)  See comments *no alloy in Subr MODEL and READ_INP'

print *,'                        press any key...'

pause

stop

endif

c

c

c find file names with the specific cross-sections from SL_LNSxx code

c (run before) for each component of the media and dE/dx from SRIM

c for projectile + media

call star(1)

Call GET_FILE_NAMES(1,NK,NF)

Do MK=1,NK

INPNAM_XS(1,MK)=NF(MK)

Enddo

*

if(istop.eq.-1) then

call star(1)

Call GET_FILE_NAMES(1,1,NF)

INPNAM_DEDX(1)=NF(1)

endif

c

c find lines with the description of material componets

Do IK=1,NK

call star(1)

read(1,*,err=90000)ZZ,AA, FR0, ED0

If(ED0.le.0.0) ED0=40.0  ! default = 40 eV

IZ=ifix(ZZ+0.001)

*

* default parameters for MD efficiency

*no alloy

If(MD.eq.1 .and. IDEFMD.eq.1) Call MDPARAM(IZ,C50)

*

ZT(IK)=ZZ

AT(IK)=AA

FR(IK)=FR0

ED(IK)=ED0 * 1.e-06    ! eV --> MeV

c

call star(1)

Call GET_FILE_NAMES(1,NK,NF)

Do MK=1,NK

INPNAM_XS(IK+1,MK)=NF(MK)

Enddo

*

if(istop.eq.-1) then

call star(1)

Call GET_FILE_NAMES(1,1,NF)

INPNAM_DEDX(IK+1)=NF(1)

endif

*

Enddo ! Do IK=1,NK

*

* find line with output file name

call star(1)

if(MD.eq.0) then

Call GET_FILE_NAMES(1,2,NF)

OUTNAM=NF(1)

OUTNAM2=NF(2)

else

* for BCA-MD get 3 file names

Call GET_FILE_NAMES(1,3,NF)

OUTNAM=NF(1)

OUTNAM2=NF(2)

OUTNAM3=NF(3)

endif

*

* find lines with energies

Do ir=1,11111111

call star(1)

read(1,*,err=2,end=2)EE,NHH

If(ir.gt.MAXE) then

print *,' T O O   M U C H   I N P U T   E N E R G I E S  !'

print *,' Change parameter MAXE in the code !'

print *,'                                 press any key...'

pause

stop

endif

EIN(ir)=ABS(EE)

If(TLIM.le.EIN(ir)) then

Print *,'           E R R O R  in INPUT file      '

Print *,'Input energy  ',EIN(ir),'  > TLIM ! '

Print *,'                      press any key...'

pause

stop

endif

*

NH(ir)=NHH

KEIN=ir

if(EE.lt.0.0) goto 2

Enddo

2      Close(1)

*

****************************************************************************

* open part of output files (units 44,55)

* (main output file is open in subr precalculation)

*

open(44,file=OUTNAM2)

If(MD.eq.0) Write(44,2000)OUTNAM

2000   Format(' See explanation of (1), (2),...',

#  ' in main output file:  "',a12,' "'/)

If(MD.ne.0) Write(44,2001)OUTNAM

2001   Format(' See explanation of (1), (3),...',

#  ' in main output file:  "',a12,' "'/)

Write(44,2002)

2002   Format(

#  ' E(MeV)         (1)       (2)',

#  '       (3)       (4)       (5)       (6)       (7)'/86('-'))

*

If(MD.eq.0) Return

open(55,file=OUTNAM3)

Write(55,2003)

2003   Format(

#  'Eff[iciency]= N(direct counting)/N(NRT)'//

#  '    T0         Eff +/- dEff  ',

#  '    N(direct) +/- dN(FR)     N(NRT)'/65('-'))

*

Return

9919   Call Error(' MD ',9919)

9929   Call Error('Z,A ',9929)

90000   print 90001

90001   Format(/'  E R R O R  in input file INPUT !!!'

#  /'  Possible reason:  dE/dx model chosen is inconsistent ',

#  /'                    with the file names introduced in INPUT'

#  /'  Note:   SPAR option - names should be omitted'

#  /'          SRIM option - names should be given')

print *,'                              press any key...'

pause

stop

End

*

************************************************************************

*

Subroutine SPAR0(ZPR, APR, E, dEdxTOTS, dEdxNS ,JJ)

*      ---------------------------------------------------

parameter (maxc=21,maxc1=22)

real *8 spar, e_proj, proj_m, z_proj, avz, at_den, bari_ln,

# fme_(maxc), a_(maxc), del_pr, sparN

*

Common/Basic/Z1,A1,Z2,A2,AW,RO,RN0

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/VENUS/RI(MAXC)

Dimension iza_(maxc)

*

*

e_proj= E

proj_m= APR*931.5d0  !    mass in MeV

z_proj= ZPR

*

avz=0.d0

bari_ln=0.d0

Do i=1,NK

avz=avz+FR(i)*ZT(i)

bari_ln= bari_ln+FR(i)*ZT(i)*alog( RI(i) )

fme_(i)= FR(i)

a_(i)  = AT(i)

IZT=ifix(ZT(i)+0.001)

IAT=ifix(AT(i)+0.001)

iza_(i)=1000*IZT+IAT

enddo

bari_ln=bari_ln/avz

*

at_den=RN0/1.d+24  ! 1/b-cm

nel1=NK

c

c

stopping=spar(e_proj,proj_m,z_proj,avz,at_den,bari_ln,fme_,a_,

# iza_,nel1,jj,del_pr,sparN)

c

dEdxTOTS=real(stopping)

dEdxNS=real(sparN)

c

Return

end

*

*

************************************************************************

*

function spar(e_proj,proj_m,z_proj,avz,at_den,bari_ln,fme_,a_,

# iza_,nel1,jj,del_pr,sparN)

*

!       REAL dedx CODING

!       dedx for heavy ion, protons,pions,muons

!       units: MeV/cm

!

!         e_proj = projectile kinetic energy (MeV)

!         proj_m = projectile mass (MeV)

!         z_proj = projectile charge number

!         avz = <Z> for the medium

!             = SUM fme_(i)*z(i)

!         at_den = atomic density (1/barn-cm)

!         bari_ln = <ln I> with I in MeV

!                 = SUM fme_(i)*z(i)*ln I(i) / <Z>

!         fme_(i)  = array of atom fracions i=1,...,nel1

!         a_(i) = array of mass numbers i=1,...,nel1

!                  (more properly, atomic weights)

!         iza_(i) = array of 1000*Z + A numbers i=1,...,nel1

!         nel1 = number of isotopes in medium

!         jj:

!            jj=1 gaseous element

!            jj=2 condensed element

!            jj=3 gaseous mixture

!            jj=4 condensed mixture

* added

!         sparN = nuclear stopping power

!

!

implicit double precision (a-h,o-z)

parameter (huge_float = 1.0d+37, tiny_float = 1.0d-37)

parameter (dp0=0.d0, dp1=1.d0, dp2=2.d0, dp3=3.d0, dp4=4.d0,

& dp5=5.d0, dp6=6.d0, dp10=10.d0, dp12=12.d0, dp24=24.d0,

& dp30=30.d0, dp41=41.d0, dp120=120.d0, dp130=130.d0,

& dp200=200.d0, dp650=650.d0, dp720=720.d0,

& dp1p30=1.d30)

parameter (dpth=dp1/dp3, dph=0.5d0, dp2th=dp2/dp3,

& dppi=3.1415926535898d0)

parameter (dp1m2=1.d-02, dp1m4=1.d-04, dp1m5=1.d-05,

& dp1m6=1.d-06, dp1m10=1.d-10, dp1m20=1.d-20)

parameter (dp4m3=4.d-03, dp2m2=2.d-02, dp9m1=0.9d0, dp11m1=1.1d0,

& dp7095m6=7.095d-03,

& dp6906m2=69.06d0)

save

parameter (prot_mass=938.2723128d0)

parameter (prot_m=prot_mass)

dimension fme_(nel1), a_(nel1)

dimension iza_(nel1)

data elow /1.31d0/, ehigh /5.24d0/

e_prot=e_proj*prot_m/proj_m

del_pr=dp0

**

stpN1=0.d0

stpN2=0.d0

**

if (e_prot.gt.elow) stp1=z_proj**2*dedx1(e_prot,proj_m,avz,at_den

& ,bari_ln,fme_(1),iza_(1),nel1,jj,del_pr)

if (e_prot.lt.ehigh) stp2=dedx0(e_proj,proj_m,z_proj,avz,at_den

& ,fme_(1),a_(1),iza_(1),nel1, stpN2)

fx=(e_prot-elow)/(ehigh-elow)

fx=min(dp1,max(dp0,fx))

c

spar=fx*stp1+(dp1-fx)*stp2

**

sparN=fx*stpN1+(dp1-fx)*stpN2

return

end

*

************************************************************************

*

function dedx0(e_proj,proj_m,z_proj,avz,at_den,fme_,a_,iza_,nel1,

&               stpN2)

!

!       dedx for heavy ion, protons,pions,muons

!

!       units: MeV/cm

!

!       low energy models from SPAR.

!       equivalent proton energy < 8 MeV.

!

!         e_proj = projectile kinetic energy (MeV)

!         proj_m = projectile mass (MeV)

!         z_proj = projectile charge number

!         avz = <Z> for the medium

!             = SUM fme_(i)*z(i)

!         at_den = atomic density (1/barn-cm)

!         fme_  = array of atom fracions i=1,...,nel1

!         a_(i) = array of mass numbers i=1,...,nel1

!                  (more properly, atomic weights)

!         iza_(i) = array of 1000*Z + A numbers i=1,...,nel1

!         nel1 = number of isotopes in medium

!

implicit double precision (a-h,o-z)

parameter (huge_float = 1.0d+37, tiny_float = 1.0d-37)

parameter (dp0=0.d0, dp1=1.d0, dp2=2.d0, dp3=3.d0, dp4=4.d0,

& dp5=5.d0, dp6=6.d0, dp10=10.d0, dp12=12.d0, dp24=24.d0,

& dp30=30.d0, dp41=41.d0, dp120=120.d0, dp130=130.d0,

& dp200=200.d0, dp650=650.d0, dp720=720.d0,

& dp1p30=1.d30)

parameter (dpth=dp1/dp3, dph=0.5d0, dp2th=dp2/dp3,

& dppi=3.1415926535898d0)

parameter (dp1m2=1.d-02, dp1m4=1.d-04, dp1m5=1.d-05,

& dp1m6=1.d-06, dp1m10=1.d-10, dp1m20=1.d-20)

parameter (dp4m3=4.d-03, dp2m2=2.d-02, dp9m1=0.9d0, dp11m1=1.1d0,

& dp7095m6=7.095d-03,

& dp6906m2=69.06d0)

save

parameter (emass=0.5109990615d0)

parameter (av_num=0.6022136736d0)

parameter (el_rad=0.28179409238d0)

parameter (stp_con=dp4*dppi*el_rad**2*emass)

parameter (prot_mass=938.2723128d0)

parameter (prot_m=prot_mass)

parameter (om30=1.0d-30)

dimension a_(nel1), fme_(nel1)

dimension iza_(nel1)

se=dp0

snuc=dp0

if (e_proj.eq.dp0) go to 20

rel_m=proj_m/prot_m

if (e_proj.gt.8.d0*rel_m) stop ' energy too large in DEDX0 '

fac1=z_proj**dpth

bcut1=.07d0*fac1**2

bcut2=.0046d0*fac1

gammasq=(e_proj/proj_m+dp1)**2

betasq=(gammasq-dp1)/gammasq

beta=sqrt(betasq)

beta0=beta

fac0=dp1

if (beta0.lt.bcut2) then

!-----

! BETA0 < BCUT2

beta=bcut2

betasq=beta**2

gammasq=dp1/(dp1-betasq)

ei=(sqrt(gammasq)-dp1)*proj_m

fac0=sqrt(e_proj/ei)

endif

a7=stp_con*avz*at_den*f_linhard(betasq,avz)/betasq

zsq=z_proj*z_proj

se=zsq*fac0*a7

if (beta0.ge.bcut1) go to 20

!-----

! Compute electronic stopping power

temp=(dp1-exp(-125.d0*beta/fac1**2))**2

se=se*temp

!-----

! Compute nuclear stopping power

do 10 j=1,nel1

z = iza_(j)/1000

q1=sqrt(fac1**2+z**dp2th)

q2=a_(j)+rel_m

q3=z_proj*z

fj=3.255d+04*a_(j)/(q2*q3*q1)

gj=1.96d-4*a_(j)*q2*q1/(q3*rel_m)

ej=fj*e_proj

if (ej.gt.1.d+03) go to 10

if (ej.gt.dp4) then

dedp=0.5455d0*log(ej)/(ej*(dp1-0.9988d0*ej**(-1.5391d0)))

else

dedp=4.46426d0*sqrt(ej)*exp(-2.542d0*ej**0.277d0)

endif

sj=(a_(j)*at_den*fme_(j)/av_num)*dedp/gj

snuc=snuc+sj

10 continue

!-----

20 continue

dedx0=max(se+snuc,om30)

**

stpN2=snuc

return

end

*

*-----------------------------------------------------------------------

...the unchanged routines of SPAR are not shown

*-----------------------------------------------------------------------

*

*

************************************************************************

*

Subroutine star(k)

*      ------------------

character c1*1

1      read(1,'(a1)',end=1000)c1

if(c1.eq.'*'.or.c1.eq.'c'.or.c1.eq.'C'.or.c1.eq.'!') goto 1

backspace 1

return

1000   if(k.eq.0) return

Print *,' U N E X P E C T E D   E N D   O F   I N P U T '

print *,'                        press any key...'

pause

stop

end

*

************************************************************************

*

Subroutine STORE_IN(MK,T,Z,IDE)

*     -------------------------------

COMMON/RECSTORE/TKIN(50003),ZZ(50003),NKO(50003),INDIC(50003),

#                 NREC

*

NREC=NREC+1

If(NREC.gt.50003) Then

Print *,'Dimension of TKIN,ZZ.. is not sufficient !'

Print *,'Check COMMON/RECSTORE/  in the code      !'

call error('TKIN',50003)

endif

TKIN(NREC)=T

ZZ(NREC)=Z

NKO(NREC) =MK

INDIC(NREC)=IDE

RETURN

END

*

************************************************************************

*

Subroutine STORE_OUT(MK,T,Z,IDE)

*     --------------------------------

COMMON/RECSTORE/TKIN(50003),ZZ(50003),NKO(50003),INDIC(50003),

#                 NREC

*

NREC=NREC-1

If(NREC.le.0) Return

T =TKIN(NREC+1)

Z =ZZ(NREC+1)

MK=NKO(NREC+1)

IDE=INDIC(NREC+1)

RETURN

END

*

************************************************************************

*

Function TDAM(IK,TkeV)

*      ------------------------

parameter (maxc1=22)

COMMON/NRT1/ANRT(MAXC1),BNRT(MAXC1),GNRT(MAXC1)

TT=TkeV

*

Tdam= TT/

# (1.+ANRT(IK)*TT+BNRT(IK)*(TT**0.75)+GNRT(IK)*(TT**0.16666666666))

RETURN

END

*

************************************************************************

*

Function TPKA(IK,MK,T0,Tmean)

*       -----------------------------

parameter (maxc=21,maxc1=22)

Common/Z2A2/ZT(MAXC),AT(MAXC),FR(MAXC),ED(MAXC),NK

Common/PKA/ALPHA2(MAXC1,MAXC),E0(MAXC1,MAXC)

Common/CUT1/rmaxpl,itot,icut

itot=itot+1

*

Emin=ED(MK)

Tmax=ALPHA2(IK,MK)*T0

If(Tmax.le.Emin) then

TPKA=Emin+0.0001

return

endif

EPS= T0/E0(IK,MK)

COEFF=ALPHA2(IK,MK)*E0(IK,MK)/EPS

*

Xmin= SQRT( Emin/COEFF )

c x(T)  X= SQRT( T0/COEFF )

*

* dx= (1/2) * (1/coeff)**0.5 / T**0.5

RM=F(Xmin)/(Emin**1.5)      ! 3/2:  1 from x**2 and 0.5 from dx

*

rn=0.

*

1000    TT=Emin+RANDOM(0)*(Tmax-Emin)

XX= SQRT( TT/COEFF )

rn=rn+1.

if(rn.gt.rmaxpl) then

icut=icut+1

TPKA=Tmean !

123      Format(1x,60('-')/1x,' IK,MK,T0,Tmean,Tmax=',2i4,3g12.5)

return

endif

1900    continue

If( RM*RANDOM(0)-F(XX)/(TT**1.5) )2000,2000,1000

*

2000    TPKA=TT

If(TT.gt.T0)then

Print *,' T0=',T0,'  Tplay=',TT,' ALPHA2=',ALPHA2

endif

*

Return

End

*

************************************************************************

*

Subroutine ZIEION(C80,Z,A)

Character C80*80

c

c Ex:   C80=' Ion = Tungsten [74] , Mass = 183.951 amu'

open(29,status='scratch')

Do i=1,80

i1=i

If(C80(i:i).eq.'[') goto 1000

enddo

Print *,' Subr. ZIEION   Symbol "[" not found !'

print *,'                           press any key...'

pause

stop

1000   Do i=i1,80

i2=i

If(C80(i:i).eq.']') goto 1001

enddo

Print *,' Subr. ZIEION   Symbol "]" not found !'

print *,'                           press any key...'

pause

stop

1001   if((i2-i1).eq.2) then

k=i1+1

write(29,'(a1)')C80(k:k)

endif

if((i2-i1).eq.3) then

k1=i1+1

k2=i1+2

write(29,'(a2)')C80(k1:k2)

endif

if((i2-i1).ne.2.and.(i2-i1).ne.3) then

Print *,' Subr. ZIEION    Error:   i2-i1=',i2-i1

print *,'                        press any key...'

pause

stop

endif

Do i=i2,80

j1=i

If(C80(i:i).eq.'=') goto 2000

enddo

Print *,' Subr. ZIEION    Second symbol "=" not found !'

print *,'                                press any key...'

pause

stop

2000   Do i=j1,80

j2=i

If(C80(i:i).eq.'a') goto 2001

enddo

Print *,' Subr. ZIEION   "amu" not found !'

print *,'                       press any key...'

pause

stop

2001   k1=j1+1

k2=j2-1

write(29,'(a12)')C80(k1:k2)

rewind 29

read(29,*)Z

read(29,*)A

close(29)

Return

end

it is supposed that Li2O consists of single Li and single O isotope.

the number depends on the amount of cards used to describe the names of files prepared by the SL_LNS4A code (see Card number 5,6 or more)